CAPE-OPEN

Delivering the power of component software

and open standard interfaces

in Computer-Aided Process Engineering

Open Interface Specifications
for Hydrodynamic Calculations
Version 0.1.008

Archival Information

| Filename | CO Hydro 0.1.007 Specification.doc |
| --- | --- |
| Authors | Martin Gainville, IFP Energies Nouvelles Gilles Ferrer, IFP Energies Nouvelles Monica Østentad , SPT Group Pierre Duchet-Suchaux, TOTAL Harald Miland, Kongsberg Jon Harald Kaspersen, Sintef Michel Pons, CO-LaN |
| Status | Internal |
| Date | May 2011 |
| Version | version 0.1.007 |
| Number of pages |  |
| Versioning |  |
| Additional material |  |
| Web location |  |
| Implementation specifications version |  |
| Comments |  |

IMPORTANT NOTICes

Disclaimer of Warranty

CO-LaN documents and publications include software in the form of sample code. Any such software described or provided by CO-LaN --- in whatever form --- is provided "as-is" without warranty of any kind. CO-LaN and its partners and suppliers disclaim any warranties including without limitation an implied warrant or fitness for a particular purpose. The entire risk arising out of the use or performance of any sample code --- or any other software described by the CAPE-OPEN Laboratories Network --- remains with you.

Copyright © 2011 CO-LaN and/or suppliers. All rights are reserved unless specifically stated otherwise.

CO-LaN is a non for profit organization established under French law of 1901.

Trademark Usage

Many of the designations used by manufacturers and sellers to distinguish their products are claimed as trademarks. Where those designations appear in CO-LaN publications, and the authors are aware of a trademark claim, the designations have been printed in caps or initial caps.

Microsoft, Microsoft Word, Visual Basic, Visual Basic for Applications, Internet Explorer, Windows and Windows NT are registered trademarks and ActiveX is a trademark of Microsoft Corporation.

Netscape Navigator is a registered trademark of Netscape Corporation.

Adobe Acrobat is a registered trademark of Adobe Corporation.

Summary

The first part of this document describes the main scenarios of usage for a Hydrodynamic Point Model component within a Unit Operation. Then in a second part, sequence diagrams describe a conceptual model that shows how a Hydrodynamic Point Model component is used in a Unit Operation.

After this analysis the design chosen is described through references to each method in the interfaces defined. Then the available properties which can be used to describe a flow are defined.

The appendices present some examples of flow patterns which define with a very fine accuracy a flow within a Unit Operation which is a pipe. These examples are useful to clarify the description of a flow in terms of layers and fields which can be difficult to understand for a Hydrodynamic Point Model non-expert.

Acknowledgements

Many individuals and their organizations have contributed to this document. Gilles Ferrer and Martin Gainville from IFP Energies Nouvelles, Monica Østentad from SPT Group, Pierre Duchet-Suchaux from Total, Harald Miland from Kongsberg Gruppen, Jon Harald Kaspersen from Sintef, Michel Pons from CO-LaN were among the main contributors.

Contents

1.	Introduction	9

1.1	Context	9

1.2	Content	9

2.	Requirements	10

2.1	Textual requirements	10

2.1.1	Layers and Fields definition and description	11

2.2	Use Cases	11

2.2.1	Actors	12

2.2.2	Use Case Priorities	12

2.2.3	List of Use Cases	12

2.2.4	Use Cases maps	13

2.2.5	Use Cases	15

2.3	Sequence diagrams	32

2.3.1	HPM Reference	32

2.3.2	HPM Validate	33

2.3.3	HPM Calculation	37

2.3.4	HPM Initialization	40

3.	Analysis and design	41

3.1	Overview	41

3.2	Sequence diagrams	41

3.3	Interface diagrams	42

3.4	State diagrams	43

3.5	Other diagrams	43

3.6	Interfaces descriptions	44

3.6.1	ICapeHydroFlow	44

3.6.2	ICapeHydroFlowRoutine	60

3.6.3	ICapeHydroPhases	66

3.6.4	ICapeHydroPropertyRoutine	70

3.6.5	ICapeHydroFields	79

3.6.6	ICapeHydroContext	90

3.6.7	ICapeHydroPackageManager	92

3.7	Scenarios	95

4.	Interface specifications	96

4.1	Interface IID and registration under the appropriate categrory	96

4.2	COM IDL	97

4.2.1	Interface ICapeHydroContext : IDispatch	97

4.2.2	Interface ICapeHydroPackageManager : IDispatch	97

4.2.3	Interface ICapeHydroPhases : IDispatch	97

4.2.4	Interface ICapeHydroFlowRoutine : IDispatch	97

4.2.5	Interface ICapeHydroPropertyRoutine : IDispatch	98

4.2.6	Interface ICapeHydroFields : IDispatch	98

4.2.7	Interface ICapeHydroFlow : IDispatch	99

5.	Notes on the interface specifications	101

5.1	Convention Note on UNDEFINED	101

5.2	Correspondance thermodynamic phase property – hydrodynamic qualifier	101

5.3	Geometry identifiers	102

5.4	Flow regime Identifiers	102

5.5	Physical Property identifiers	103

5.5.1	Single-phase properties	103

5.5.2	Two-phase properties	103

5.6	Hydrodynamic Flow Properties	104

5.6.1	Mixture properties	104

5.6.2	Single Phase properties	104

5.6.3	Two-phase properties	104

5.7	Hydrodynamic Layer and Field properties	104

5.7.1	Layer pattern identifiers	104

5.7.2	Single-layer properties	105

5.7.3	Two-layer properties	105

5.7.4	Single-field properties	105

5.7.5	Phase properties	106

6.	Prototype implementation	107

6.1	Example of flow calculation with phase mixing rule done by <Unit Operation>	107

6.2	Example of flow calculation with phase mixing rule done internaly in HPM	108

6.3	Example of field retrieve property	109

7.	Specific glossary terms	110

8.	Bibliography	111

8.1	Process simulation references	111

8.2	Computing references	111

8.3	General references	111

9.	Appendices	112

9.1	Example of 2-phase flow patterns defined with Layers and Fields	112

9.2	Examples of 3-phase flow patterns defined with Layers and Fields	113

List of Figures

Figure 1 HPM Package Configuration Use Cases	13

Figure 2 HPM Package Context Use Cases	13

Figure 3 HPM Package Calculation Use Cases	14

Figure 4 HPM reference to a Unit Operation when selected in the PME	32

Figure 5a Validation of HPM number of recognized phases	33

Figure 5b Validation of HPM specifications and output properties	36

Figure 5c Validation for HPM geometry and thermodynamic properties inputs	37

Figure 6a HPM Calculation sequence (HPM mixing rule)	38

Figure 6b HPM Calculation sequence (UO mixing rule)	39

Figure 7 HPM Initialisation	40

Type of Data	101

Declaration and Usage	101

Thermodynamic	101

Figure 8 Annular flow regime defined by layers and fields	112

Figure 9 Slug flow regime defined by layers and fields	114

Figure 10 Slug flow regime with a stratified bed defined by layers and fields	116

# Introduction

## Context

For process modeling, the CAPE-OPEN standard has established programming rules and communication protocols for integrating various types of software components such as thermodynamic components and unit operations into a process modeling environment. Now major process modeling environments are CAPE-OPEN compatible and consequently offer open and flexible architectures to plug and play with process modeling components.

Since 2004, upstream applications and studies have used the CAPE-OPEN standard to simulate oil and gas production systems that require advanced thermodynamic calculations [1][2]. In these applications, pipe modules are widely used to model flows in wells, flow lines and production risers. In addition to advanced thermodynamic calculations, pipe flow models require some multiphase flow correlations for predicting phase distribution and slip velocities between present phases. These correlations are more or less complex and are often derived from literature publications and university laboratory works before being found in commercial software products.

Hydrodynamic correlations are also used for characterizing phase slippage, for example for modeling flow through a valve throat. In a way, the hydrodynamic concept can be applicable to any unit operation that requires specific hydrodynamic calculations.

Under these considerations, there is an opportunity to define a hydrodynamic calculation interface standard that specifies communications between hydrodynamic module software components, process modeling environments and unit operations.

## Content

This document defines conceptual interfaces and usages for dealing with hydrodynamic calculations in a context of fluid flow within a flowsheet environment.

A Hydrodynamic Point Model (hereafter called HPM) is a model that predicts pressure drop, phase holdups, phase velocities and possibly other flow variables for a cross section at one point of a given piece of equipment. A HPM may cover a large domain of applications. It is dedicated to flows through pipe/well sections and also may be applied to flows through a valve or an annular space. The "point" – part of the name hence refers to the direction along the pipeline, i.e. normal to the cross section. Current commercial HPMs (OLGA-S [3], TACITE [4]) deliver average flow values for cross section or for pipe unit element in case of intermittent flows. Future versions may in addition predict cross sectional distributions of some variables, for instance velocity profiles and concentration profiles.

In this first iteration step of the interface design, we will focus the application domain to multiphase flow in a pipe. It means that we will focus on defining relationships between an HPM calculation server and clients which can be either a pipe module present as a Unit Operation, or the Process Modelling Environment (hereafter called PME) or the End-User.

The HPM is only in interaction with a Unit Operation; it is the responsibility of the PME and the End-User to associate the suitable HPM with a given Unit Operation: protocols are established to fulfil this requirement.

The recommendations of the Conceptual Design Document [6] within the CAPE-OPEN documentation are used. In the Analysis phase, user requirements are described and expressed as Use Cases.

# Requirements

We formalise the description of the user requirements for Hydrodynamic Point Model (HPM) and the relationships with its different Clients: pipe module as Unit Operation, End-User and the PME. In the following, when Client is used as singular it refers to any of the different Clients defined above. Elsewhere a specific Client may be mentioned for a given requirement.

## Textual requirements

Possibility to configure HPM is a user requirement. For instance, the End-User may need to select a hydrodynamic correlation from several possible ones or may need to specify emulsion viscosity curves that have to be taken into account during calculations.

End-User may also need the possibility to tune the HPM, for instance by modifying friction coefficients or some other model parameters. The tuning algorithm is considered to be outside the HPM: it means that there is a requirement for a Parameter Collection to be displayed by the HPM in order to give access to its model parameters.

A HPM calculation provides calculation results to its Client. A set of standard hydrodynamic results can be defined that will be delivered by any HPM calculation: pressure drop, phase velocities, phase holdups and flow regime. The main purpose of any HPM is to get these properties calculated. Hence, for efficiency purposes, a user requirement is to get a dedicated access by a Client to this minimum set of properties.

For performing calculation, a HPM can internally use a set of phases different from the phases given by the thermodynamic module. For example the HPM can use only gas and liquid phases in case of two-phase computation, or gas, oil and water in case of three phases computation. A HPM can have internal mixing rule to transform thermodynamic phases to "hydrodynamic" phases (remark: OLGA-S and TACITE hydro have not this implementation at the moment and mixing rule management is mainly the responsibility of the client. For OLGA-S the client needs to define two-phase or three-phase data depending on which method is called, for TACITE two-phase hydrodynamics data can be provided in three phases or twp phases and in case of three-phase data mixing rules are performed internally. In case of more than three phases, TACITE does not manage the mixing). Some Pipe Unit Operations can also propose a mixing rule to transform the thermodynamic phases in a subset of phases. The Unit Operation (Client of the HPM) should have the possibility to validate if the input HPM phases are compatible with the output thermodynamic phases.

The possibility for a Client to impose the flow regime before performing a calculation is another requirement. In this case, the final results of the HPM could be different from the results obtain with the HPM general settings that may allow for flow regime determination by the HPM.

Some additional hydrodynamic property calculations may be decoupled from the general hydrodynamic calculation. For instance, the gas/liquid shear stress can be calculated for given phase holdups and velocities and the Client may want to calculate the property for given inputs.

A validation scheme process is needed to check the validity of a HPM. In this process, the HPM may confirm to its Client that its configuration has been correctly done.

Analysis of existing HPMs shows that they may slightly differ in the list of input variables they need and output variables they deliver. A HPM has to provide information to Clients about input requirements and properties available as output. With these properties labels, a Client can deliver the requested information to the HPM which conserves flexibility in its inputs definition (not all HPMs need to have the same inputs definition).

A set of internal parameters of HPM should be possibly stored and then reloaded for initialising HPM algorithm calculations in a further calculation step. This option may be convenient for minimizing calculation time.

The HPM can propose the access to internal information: layers and fields. The layers and fields concept offer a detailed description of flow configurations; for instance it can be used for characterizing emulsion properties, intermittent flow...

### Layers and Fields definition and description

Layers and Fields are entities which described precisely the flow configuration.

A layer is a continuous area in term of composition and properties. Properties can include dispersion. A field is a flow entity where present phases flow at the same average velocity. A field can be an emulsion or not. Layers and Fields are referenced by a unique and constant  label.

The complete flow is described by a set of layers composed of fields. Each thermodynamic phase may be distributed in several fields. The sum of a given phase over present fields gives back the entire phase.

A layer is characterized by a set of fields and one of these fields is corresponding to the continuous field. One can make access to layer properties, i.e. interface perimeters, wetted perimeter,..., and configurations, i.e. stratified or annular, dispersed or not... Regarding the layer definition, a layer with more than one field should be used to define a dispersed flow.

A field is characterized by present phases, the continuous phase (one of the present phases), and the volume fractions in the field. Some properties can be accessed, i.e. field velocity, emulsion state. Regarding the field definition, a field with more than one phase should be used to define an emulsion state.

Examples of flow patterns described in term of layers and fields are presented in section 9.1: annular gas and water flow; slug gas, oil and water flow.

## Use Cases

The standard defines the following primary CAPE-OPEN components:

- HPM Package <HPM Package> A software component that calculate hydrodynamic properties: pressure drop, phase holdups, velocities...

- HPM Package Manager <HPM Package Manager> A software component that manages a set of <HPM Package>. It is responsible for instantiating <HPM Packages> on request and may allow <HPM Packages> to be edited.

The main purpose of a <HPM Package Manager> is to allow a developer to implement the instantiation of a <HPM Package>. As a primary CAPE-OPEN component, a <HPM Package manager> may allow a client to edit <HPM Packages> and create new ones.

A <HPM Package Manager> is responsible for managing a set of <HPM Packages>. It implements the interface which allows a client to get the list of the names of the <HPM Packages> managed by the component and to request that a <HPM Package> be instantiated.

### Actors

- End User <End User> The physical person who asks the Unit Operation to make use of a given <HPM Package> for calculating pressure drop and flow properties. The End-User will optionally configure the selected <HPM Package>.

- Process Modelling Environment <PME> The <PME> makes use of components, CAPE-OPEN or not, in order to model the flow within a flowsheet. In CAPE-OPEN documents the synonym COSE can be used for <PME>.

- Unit Operation <Unit Operation> [8] The component uses a <HPM Package> to calculate hydrodynamic properties. For this purpose, it checks the validity of the <HPM Package> and provides the <HPM Package> with the required pieces of information. In a first iteration step, we assume that a <Unit Operation> corresponds to a pipe.

### Use Case Priorities

- High. Essential functionality for an HPM Package. Functionality without which the operation usability or performance of an HPM Package might be seriously compromised

- Medium. Very desirable functionality that will make HPM Packages more usable, transportable or versatile. The essence of an HPM Package is not compromised by this Use Case, although the usability and acceptance of the component can be.

- Low. Desirable functionality that will improve the performance of HPM Packages. If this Use Case is not met, usability or acceptance can decrease.

### List of Use Cases

- UC-001: Load HPM Package

- UC-002: HPM Package Configuration

- UC-002A: Configure HPM Package

- UC-002B: HPM Package specific data

- UC-003: HPM Package association

- UC-003A: Check Unit Operation compatibility

- UC-003B: Reference HPM Package

- UC-004: Copy HPM Package

- UC-005: Delete HPM Package

- UC-006: HPM Package validation

- UC-007: Hydrodynamic calculation

- UC-008: Impose flow regime

- UC-008A: Impose locally flow regime

- UC-008B: Impose alternative flow regime

- UC-009: Hydrodynamic specific property calculation

- UC-010: Input properties

- UC-011: Delivered properties

- UC-011A: Phase delivered properties

- UC-011B: Field delivered properties

- UC-012: Calculation initialization

- UC-013: HPM Package tuning

### Use Cases maps

Figure 1 HPM Package Configuration Use Cases

Figure 2 HPM Package Context Use Cases

Figure 3 HPM Package Calculation Use Cases

### Use Cases

#### UC-001: Load HPM package

Actors: <PME> - <Unit Operation> - <End User>

Priority: <High>

Classification: <Hydro Use Case> - <HPM Package Configuration>

Context:

Several <HPM Package>s are available for the <PME>. The <End User> selects one within the list to be loaded in the <PME>.

Pre-conditions:

A <HPM Package Manager> is available in the <PME> for selecting the <HPM Package>.  The <HPM Package Manager> can be part of the <PME> or a third party component available for the <PME>.

Flow of events:

Through the HPM Package Manager, the <PME> gets the list of the CAPE-OPEN compliant <HPM Package>s available. The <End User> selects one <HPM Package> and the HPM Package Manager creates an instance available for the <PME>.

A <Unit Operation> can implement this Use Case to select a locally specific <HPM Package>. The selected  <HPM Package> is available only to the current <Unit Operation> and replaces the <HPM Package> possibly defined in a previous step.

At the end, the HPM Package Manager can be destroyed.

Post-conditions:

A <HPM Package> instance is defined as available to the Client. It is the responsibility of the Client to check the validity of the <HPM Package>.

Errors:

Uses:

Extends:

#### UC-002: HPM Package configuration

The Use Cases of <HPM Package> configuration can be exercised either at the <PME> level, when the <HPM Package> is provided to any <Unit Operation>, or at the <Unit Operation> level, when the <HPM Package> is provided for a specific <Unit Operation>.

#### UC-002A: Configure HPM package

The CAPE-OPEN standard does not define how the configuration stage should be performed for any CAPE-OPEN component. This Use Case is defined to inform the reader what can be a <HPM Package> configuration stage. This Use Case can be a part of a stand-alone software.

Actors: <End User>

Priority: High

Classification: <Hydro Use Case> - <HPM Package Configuration>

Context:

A <HPM Package> may be set in different configurations to compute the pressure drop and other flow variables. For instance, the 2 or 3 phase choice may be selected at this configuration stage, as well as the selection of specific correlations or options. Another configuration capacity may concern emulsion viscosities; the <HPM Package> may use default viscosities provided by its calculation client, combined with default internal mixing rules, or use emulsion curves defined at the configuration stage.

Pre-conditions:

[UC-001A Load HPM Package] has been realized.

Flow of events:

The <End User> edits the <HPM Package>. The simulation options and parameters are defined for the <HPM Package>, i.e. selection for a 2 or 3 phase option, hydrodynamic correlations used, emulsion viscosity curves... At the end of this configuration stage, a configure <HPM Package> is available for the <PME> client. The <HPM Package> may provide a functionality to create a new <HPM Package> based on the modified one.

Example of specific parameters for a <HPM Package> configuration (from OLGAS interface): water droplet entrainment multiplier, water fraction at inversion point, switch for oil  water slip calculation, input slip velocity oil/water, switch for turning on/off dispersion viscosity calculation, switch for turning on/off water correlation for air/water low pressure experiments.

Post-conditions:

A configure  <HPM Package> is defined as available for the <PME>.

Errors:

Uses:

Extends:

#### UC-002B: HPM package specific data

Actors: <End User> - <PME>

Priority: High

Classification: <Hydro Use Case> - <HPM Package Configuration>

Context:

This Use Case is fulfilled by the "Edit" or "GetParameters" methods in the CAPE-OPEN interfaces.

Pre-conditions:

A <HPM package> set of specific parameters values can be specified.

Flow of events:

The <End User> asks the <PME> to edit the <HPM Package>. The <PME> attempts to invoke the "Edit" method on the <HPM Package>.

For <HPM Package> which have not proprietary user interface, the <PME> should allow the <End User> to change the values of  parameters

Post-conditions:

The <HPM Package> parameters values have been specified.

Errors:

<Failed to access value> data values cannot be specified.

Uses:

Extends:

#### UC-003: HPM Package Association

#### UC-003a: check Unit Operation Compatibility

Actors: <PME> - <Unit Operation>

Priority: High

Classification: <Hydro Use Case> - <HPM Package Context>

Context:

Not all <Unit Operation>s have to support the <HPM Package> interfaces. Before referencing a <HPM Package> to a <Unit Operation>, the <PME> valid the <Unit Operation> is <HPM Package> compatible.

Pre-conditions:

A new <HPM Package> instance or <Unit Operation> is present on the <PME>.

Flow of events:

Before associating the <HPM Package> to the <Unit Operation>, the <PME> needs to know if the <Unit Operation> is compatible with <HPM Package> interfaces. A compatible <Unit Operation> exposes a specific hydrodynamic interface that permits the association. The <PME> asks the <Unit Operation> for the presence of this interface.

Example for a COM software component: the <PME> asks a "QueryInterface" to the <Unit Operation> using the IID of the <HPM Package> interface.

Post-conditions:

Errors:

Uses:

Extends:

#### UC-003b: reference HPM package

Actors: <PME> - <Unit Operation>

Priority: High

Classification: <Hydro Use Case> - <HPM Package Context>

Context:

The <PME> allows the <Unit Operation> that implements the HPM Package interface to access the <HPM Package>.

Pre-conditions:

A <HPM Package> instance is present in the <PME> and the <Unit Operation> is <HPM Package> compatible.

Flow of events:

A <Unit Operation> is defined in the flowsheet. The <PME> associates the <HPM Package> instance to the <Unit Operation>.

Remark: the <HPM Package> instance may be local to the <Unit Operation>. It is the <Unit Operation> responsibility to create a local copy of the referenced <HPM Package>, and delete it.

If the [UC-001 Load HPM package] is realized after defining a <Unit Operation>, the connection is done for all the compatible <Unit Operation>s present in the flowsheet and implementing the required interface.

If a local <HPM Package> is present in the <Unit Operation> and the <PME> sends a new one, it is the responsibility of the <Unit Operation>  to decide the replacement of the previous one.

For <Unit Operations> which use a <HPM Package> selected through the <PME>, if the <PME> asks for the reference event, the previous <HPM Package> is replaced.

Post-conditions:

A <HPM Package> instance is present on the <Unit Operation>.

Errors

<Failed to connect HPM Package> the connection interface is present on the <Unit Operation> but "not" implemented (for example if the Pipe Unit Operation uses its own hydrodynamic module than can not be overloaded).

Uses:

Extends:

#### UC-004: Copy HPM Package

Actors: <PME>

Priority: High

Classification: <Hydro Use Case>

Context:

Each <Unit Operation> use a specific instance of <HPM Package>. The <PME> has to create the copy before associating a <HPM Package> to a <Unit Operation>.

Pre-conditions:

A <HPM Package> instance is present in the <PME>.

Flow of events:

A new <HPM Package> is created.

The <PME> calls the <HPM Package> to generate a copy from the previous one. The new <HPM Package> copies all parameters values and properties.

Post-conditions:

Two identical <HPM Package> are present on the <PME>.

Errors:

<Failed Operation> A copy cannot be created due to an unauthorized licence problem.

Uses:

Extends:

#### UC-005: Delete HPM package

Actors: <End User> - <PME> - <Unit Operation>

Priority: High

Classification: <Hydro Use Case>

Context:

<HPM Package> are deleted by <PME> or <Unit Operation>

Pre-conditions:

Flow of events:

The <End User> requests the <PME> deletes the selected <HPM Package>.

The <PME> asks for the "Terminate" method to the <HPM Package>. The <PME> then deletes the <HPM Package> instance and zeroes all .references to the <HPM Package>.

Post-conditions:

<HPM Package> is deleted and no references are valid.

Errors:

Uses:

Extends:

#### UC-006: HPM Validation

Actors: <Unit Operation>

Priority: High

Classification: <Hydro Use Case> - <HPM Calculation>

Context:

Before flow calculation, a <HPM Package> should be validated: the necessary input properties should be available in the <Unit Operation>, calculated output properties should comply with expected <Unit Operation> properties.

Pre-conditions:

A reference to a <HPM Package> is available to the <Unit Operation>.

Some thermodynamic methods are present on the <Unit Operation> to obtain information about lists of defined phases and properties.

Flow of events:

The validation stage is divided in three main steps: validation of the <HPM Package> calculation context, validation between <Unit Operation> and <HPM Package> properties, validation between thermodynamic properties and <HPM Package> properties.

The <Unit Operation> checks if the calculation context (i.e. flow calculation using phases superficial velocities as input) is available.

To compute flow properties, a <HPM Package> will need to obtain a number of geometric properties from the <Unit Operation>. The <Unit Operation> asks the <HPM Package> which geometric properties are needed, and validates if these properties are available. The <Unit Operation> verify if the required output flow properties will be available from the <HPM Package> after the calculation.

In addition, the <HPM Package> needs a number of thermodynamic properties. Not all <HPM Packages> need the same properties. The <Unit Operation> asks which input properties will be required, and validates if the properties are available. Not all <HPM Packages> accept any thermodynamic phases. The <Unit Operation> asks the <HPM Package> if it is a full or restricted phases <HPM Package> (example of restricted <HPM Package>: two phases gas-liquid HPM). If the <HPM Package> is a restricted <HPM Package>, the <Unit Operation> needs to have internal phases mixing rule compatible with the input phases accepted by the <HPM Package>.

Post-conditions:

Errors:

<Failed to validate>: some information are not present for validation

Uses: [UC-010 Input properties] – [UC-011 Delivered properties]

Extends:

#### UC-007: Hydrodynamic Calculation

Actors: <Unit Operation>

Priority: High

Classification: <Hydro Use Case> - <HPM Package Calculation>

Context:

The <Unit Operation> needs a hydrodynamic calculation to resolve the conservative equations for a specific geometry, a set of thermodynamic phases and their related properties and a given set of flow parameters.

Pre-conditions:

A validated <HPM Package> is present for the <Unit Operation>. Input data for performing a <HPM Package> calculation are available. The number of present thermodynamic phases and state of aggregation are known.

One may distinguish at least three types of <HPM Package> calculation depending on flow parameters specification:

1. Set of phase superficial velocities,

2. Set of phase holdup and overall mixture velocity,

3. Set of phase holdup and overall barycentric velocity.

Flow of events:

The <Unit Operation> sends input data to the <HPM Package>: geometry, thermodynamic phases and relative properties, and flow parameters and asks for calculation.

The calculation determines overall pressure drop, phase holdups, velocities of present thermodynamic phases and flow regime. Optionally, composition and properties of present hydrodynamic fields may be calculated.

At the end of the calculation, the standard properties are available for retrieval from the <HPM Package> by the <Unit Operation>.

Post-conditions:

Overall pressure drop, phase holdups, phase velocities, flow regime are available to the <Unit Operation>.

Errors:

<Failed to access value> a needed input value is not accessed.

<Failed to calculate>

Uses:

Extends:

#### UC-008: Impose flow regime and Calculate

#### UC-008A: Impose locally flow regime

Actors: <Unit Operation>

Priority: Medium

Classification: <Hydro Use case> - <HPM Calculation>

Context:

The <Unit Operation> wants to calculate the hydrodynamic properties with a specified flow. The flow regime identifiers have to be known by the <Unit Operation>.

Pre-conditions:

The <End User> has specified to the <Unit Operation> which flow regime to consider. For instance, an imposed flow regime profiles may be defined by the <End User> along a pipe during the configuration stage of the pipe.

Flow of events:

The <Unit Operation> sends the nature of the flow regime to consider in addition to the type of calculation requested [UC-004 Hydrodynamic calculation]. The <HPM Package> performs hydrodynamic calculation [UC-004] using the imposed flow regime.

Post-conditions:

After the calculation, the imposed flow regime is turned to "Undefined".

Errors:

<Failed to access value> some necessary input value is not accessible

<Failed to calculate>

Uses: [UC-007 Hydrodynamic calculation]

Extends:

#### UC-008b: Impose alternative flow regime

Actors: <Unit Operation>

Priority: Medium

Classification: <Hydro Use case> - <HPM Calculation>

Context:

A hydrodynamic calculation without imposes flow regime has been realized and return an output error. A new calculation is performed with an imposed flow regime.

Pre-conditions:

Flow of events:

A hydrodynamic calculation [UC-004] is performed and return the <Failed to calculate> error. The <Unit Operation> specifies to the <HPM Package> which flow regime is to be used for the alternative calculation. The <HPM Package> performs the hydrodynamic calculation with this new specification.

For example, the specified flow regime can be a flow regime obtained during a previous calculation.

Post-conditions:

After the calculation, the imposed flow regime is turned to "Undefined".

Errors:

<Failed to access value> a necessary value is not available

<Failed to calculate>

Uses:

Extends: [UC-007 Hydrodynamic calculation] - [UC-008A Impose locally flow regime]

#### UC-009: Hydrodynamic specific property calculation

Actors: <Unit Operation>

Priority: High

Classification: <Hydro Use Case> - <HPM Calculation>

Context:

The <Unit Operation> needs a list of hydrodynamic properties to perform its calculation scheme. It asks the <HPM Package> to perform the property calculation by specifying the property name on the desired phase/field (or two-phase/fields in case of two field property).

Pre-conditions:

A [UC-007 Hydrodynamic calculation] or [UC-008 Impose flow regime] has been previously performed.

Remark: the flow calculation can not be realized through this sequence.

Flow of events:

The <Unit Operation> asks the <HPM Package> to perform the calculation of a property or a list of properties for a set of given phase/fields. The <HPM Package> performs the calculation.

Post-conditions:

The property value or the list of property values is available to the <Unit Operation> for retrieval.

Errors:

<Failed to calculate property> the calculation for a property failed.

<Not provided property> a property is not provided by the <HPM Package>.

<Not present phase>

Uses:

Extends:

#### UC-010: Input properties

Actors: <Unit Operation>

Priority: High

Classification: <Hydro Use Case> - <HPM Calculation>

Context:

<HPM Package>s differ in their list of input properties needed to perform a given calculation. There is a need to know these properties to check the <HPM Package> validity within the <Unit Operation>.

Pre-conditions:

A <HPM Package> instance is available for the Pipe <Unit Operation>.

Flow of events:

<Unit Operation> asks to <HPM Package> the list of input properties needed to perform a calculation specifying the calculation context (see [UC-0047).  The input properties are geometric properties, single-phase or two-phase thermodynamic properties.

The <HPM Package> returns a list of property identifiers to be supplied by the <Unit Operation>.

Post-conditions:

<Unit Operation> knows input properties used by <HPM Package>.

Errors:

Uses:

Extends: [UC-006 HPM validation]

#### UC-011: Delivered properties

Delivered properties can be asked per phase or per field. All <HPM Packages> need to deliver output phases properties. Output field properties are optional.

#### UC-011a: Phase delivered properties

Actors: <Unit Operation>

Priority: High

Classification: <Hydro Use Case> - <HPM Calculation>

Context:

The <Unit Operation> needs the list of supported properties calculated by the <HPM Package> in order to ask the <HPM Package> only for properties that can be calculated and to check if the <HPM Package> fulfils the needs of the <Unit Operation> in terms of properties calculations.

Pre-conditions:

Flow of events:

The <Unit Operation> asks to <HPM Package> the list of supported properties.

The <HPM Package> returns the list of properties identifiers. These properties are mixture, single phase or interface properties. The available derivative properties are also returned.

Post-conditions:

Errors:

Uses:

Extends: [UC-006 HPM validation]

#### UC-011b: Field delivered properties

Actors: <Unit Operation>

Priority: Medium

Classification: <Hydro Use Case> - <HPM Calculation>

Context:

The <Unit Operation> needs the list of layers and fields calculated by the HPM Package. These layers, fields and properties are  parameters of the <HPM Package> and are available for additional flow description to the Client.

Pre-conditions:

HPM calculation as been performed.

Flow of events:

The <Unit Operation> asks to <HPM Package> the list of present layers labels and fields labels from the layers.

The <Unit Operation> can ask the layer and field information and properties corresponding to these labels.

Post-conditions:

Errors:

<Not implemented function> same meaning as no layer and field properties delivered

Uses:

Extends:

#### UC-012: Calculation Initialisation

Actors: <Unit Operation>

Priority: Low

Classification: <Hydro Use Case> - <HPM Calculation>

Context:

It can be suitable to initialise some internal variables of a <HPM Package> to improve computational performance. Some pieces of information previously stored are sent to the <HPM Package> before performing a new calculation. An anonymous data vector is used to ensure storage and initialisation of the <HPM Package>.

Pre-conditions:

An initialisation data vector previously stored is available after the <HPM Package> first calculation. For the first calculation, an empty vector can be sent.

Flow of events:

For the <HPM Package> first calculation, the <Unit Operation> sends an empty data vector and asks for <HPM Package> calculation.  After the calculation, an initialisation data vector is available on the <HPM Package>. The <Unit Operation> requests and stores the vector.

For next calculations, the sequence is the same with the previous initialisation data vector.

Remark:  As an anonymous data vector, the <Unit Operation> can not perform any action on it; no data usage or modification can be done. If the data vector is empty, the <HPM Package> will use its internal initialisation values to perform the calculation (this is always the case for the first calculation). If the data vector contains values, the <HPM Package> will use it to initialise the calculation. After the calculation, a set of modified values in the data vector is available for the <Unit Operation>. If the <HPM Package> calculation fails, the available data vector still remain the input one

The data vector is anonymous, this mean that the <Unit Operation> does not have the description of the vector dimension and the values inside. The <Unit Operation> only keeps and sends it to initialise the <HPM Package> calculation. An implementation of the vector can specify a dedicated <HPM Package> key in the first value item for example which can be verified during the retrieval operation. This is a proposal of implementation which is not imposed by the standard.

Post-conditions:

A data vector resulting from calculation is available for the <Unit Operation>.

Errors:

<Initialisation error > the data vector is not compatible with the <HPM Package>.

Uses:

Extends: [UC-007 Hydrodynamic calculation]

#### UC-013: HPM Tuning

Actors: <End User> - <PME>

Priority: Low

Classification: <Hydro Use Case> - <HPM Calculation>

Context:

Tuning a <HPM Package> can be useful for <End User> in relation to experiments..

Pre-conditions:

A <HPM Package> tuning method is available in the <PME>.

Flow of events:

The <PME> asks for the parameter collection using the "GetParameters" method. The <PME> changes some parameters values and asks for a new calculation. The <PME> asks for results and the <End User> decides to iterate in the flow of events if necessary.

Post-conditions:

Errors:

<Failed to access value> parameters values cannot be accessed.

Uses:

Extends:

## Sequence diagrams

### HPM Reference

Figure 4 describes the sequence diagram for a <HPM Package> implemented as a COM component which adds a valid <HPM Package> reference to a Unit Operation.

First, the <PME> asks the <Unit Operation> if the corresponding interface is present. If it is present, the <PME> send the HPM Package to be referenced by the <Unit Operation>.

Figure 4 HPM reference to a Unit Operation when selected in the PME

Before the add operation, the <Unit Operation> should verify if the <HPM Package> is compatible with its internal specifications (HPM Validate 2.3.2)

For example, verify the <HPM Package> delivers all necessary hydrodynamic properties to realize the <Unit Operation> calculation, <HPM Package> input geometric and physical properties are defined in the <Unit Operation>... These validations can be performed even if the thermodynamic Material Object is not present on the <Unit Operation>.

If the <HPM Package>-<Unit Operation> compatibility is validated, the <HPM Package> can be added to the <Unit Operation>; if an error occurred, the <HPM Package> is not added and the <Unit Operation> cannot perform its own calculation.

### HPM Validate

#### HPM compatibility validate

A <HPM Package> can present different specifications defined by the accepted input phases. Either a <HPM Package> is defined as an "all phases" <HPM Package> (it accepts all defined thermodynamic phases and generates hydrodynamic properties for all these phases), either it is a "restricted" phases <HPM Package> (for example, the <HPM Package> accepts only two phases: gas and liquid).

A validation sequence needs to verify the compatibility on phase identifiers between the <HPM Package> and the <Unit Operation>. If the <Unit Operation> does not contain internals mixing rules, only all phases <HPM Package> may be compatible. For “restricted” phases <HPM Package>, the <Unit Operation> needs to perform mixing rules before calling the <HPM Package> calculation. In this case, the <Unit Operation> verifies that the internal phase identifiers comply with the ones used by the <HPM Package>. For example a <Unit Operation> which conserves the solid phases cannot be linked with a gas-liquid <HPM Package>.

Figure 5a Validation of HPM number of recognized phases

A phase label needs to be associated to a standardized hydrodynamic phase identifier. This equivalence is done in the <Unit Operation> by using the ICapeThermoPhases::GetPhaseInfo method. An identifier can be present more than once in the present thermodynamic phases.

The correspondence between thermodynamic phase informations and hydrodynamic phase identifiers is defined in the present table. These specifications are done from the [Thermodynamic and Physical Properties v1.1] document for thermodynamic properties.

| CAPE-OPEN 1.1 Thermodynamic |  | Hydrodynamic phase description |  |
| --- | --- | --- | --- |
| StateOfAggregation |  | StateOfAggregation |  |
| Vapor |  | Vapor |  |
| StateOfAggregation | KeyCompoundId | StateOfAggregation | Type (OfLiquid) |
| Liquid | UNDEFINED | Liquid | Organic |
|  | Water |  | Aqueous |
|  |  |  | UNDEFINED |
| StateOfAggregration | TypeOfSolid | StateOfAggregation | Type (OfSolid) |
| Solid | PureSolid | Solid | PureSolid |
|  | SolidSolution |  | SolidSolution |
|  | HydrateI |  | Hydrate |
|  | HydrateII |  | Hydrate |
|  | HydrateH |  | Hydrate |
|  |  |  | UNDEFINED |
| Unknown |  | Unknown |  |

- Note for Unknown stateOfAggregation: this state of aggregation value appears according to the Thermodynamic specification. In most case, it will be the responsibility of the <Unit Operation> or of the <HPM Package> to define an hydrodynamic correspondence or to raise an error if no correspondence is available.

Example of correspondence for a thermodynamic 5 phase composition to a N-Phase hydrodynamic module. All the thermodynamics phases information have a corresponding hydrodynamic identifier, .

| Thermodynamic |  |  | Hydrodynamic |  |
| --- | --- | --- | --- | --- |
| Phase label | StateOfAggregation | KeyCompoundId TypeOfSolid | StateOfAggregation | Type (OfSolid) Type (OfLiquid) |
| "phaseA" | Vapor |  | Vapor |  |
| "phaseB" | Liquid | UNDEFINED | Liquid | Organic |
| "phaseC" | Solid | HydrateI | Solid | Hydrate |
| "phaseD" | Liquid | Water | Liquid | Aqueous |
| "phaseE" | Solid | HydrateII | Solid | Hydrate |

Correspondence with the same thermodynamic 5 phases composition to a "restricted" 3-Phase (Vapor, Liquid, Solid) hydrodynamic module. Thermodynamic phase properties will be mixed by the hydrodynamic module.

| Thermodynamic |  |  | Hydrodynamic |  |
| --- | --- | --- | --- | --- |
| Phase label | StateOfAggregation | KeyCompoundId TypeOfSolid | StateOfAggregation | Type (OfSolid) Type (OfLiquid) |
| "phaseA" | Vapor |  | Vapor |  |
| "phaseB" | Liquid | UNDEFINED | Liquid | UNDEFINED |
| "phaseC" | Solid | HydrateI | Solid | UNDEFINED |
| "phaseD" | Liquid | Water | Liquid | UNDEFINED |
| "phaseE" | Solid | HydrateII | Solid | UNDEFINED |

#### HPM delivered properties validate

Different HPM calculation contexts, i.e. [Us][], [Rk][Um], can be realized in a <Unit Operation>. The <Unit Operation> needs to validate the required calculations are present on the <HPM Package>, and then the required flow properties will be available in these contexts.

Figure 5b Validation of HPM specifications and output properties

#### HPM input properties validate

Compute flow calculation depends on the geometry of <Unit Operation>. The <Unit Operation> validates that the required geometric properties are defined.

A <HPM Package> needs some thermodynamic properties as inputs to compute the flow properties. One can verify if these properties are supported by the <Material Object>. To be valid, a <HPM Package> should use input thermodynamic properties which are a subset of the supported thermodynamic properties.

Figure 5c Validation for HPM geometry and thermodynamic properties inputs

### HPM Calculation

Figure 6 shows a Sequence Diagram for a HPM calculation [UC-003]. To simplify the Sequence Diagrams, we do not present the way to calculate thermodynamic properties which are assumed to be available at this stage. The <Unit Operation> needs to check the relationship between thermodynamic phase labels and hydrodynamic supported phases using the correspondence table previously defined

Depends on the <HPM Package>, two implementations can be realized for calculation:

- In the first one, the <Unit Operation> converts thermodynamic phase labels to hydrodynamic identifiers. Then, the <Unit Operation> sends all phases input data to the <HPM Package>. If the HPM calculation uses a restricted number of phases, it is in charge to convert input data to internals data with its own mixing rule to realized the calculation.

- In the second one, the <HPM Package> is a "strict" number of phases <HPM Package>, without internal mixing rules to convert input data. For example, we define a sequence diagram with a <HPM Package> which uses two hydrodynamic phases: gas and liquid. The <Unit Operation> is in charge to send input data according to hydrodynamic identifiers accepted by the <HPM Package>. In this case, the <Unit Operation> needs to have a mixing rule to realize this operation.

.

Figure 6a HPM Calculation sequence (HPM mixing rule)

Figure 6b HPM Calculation sequence (UO mixing rule)

In the second diagram, a <HPM Package> which accept all thermodynamic phases can be link to the <Unit Operation>. In that case, the <Unit Operation> mixing rule will not have any effect to input data.

### HPM Initialization

A HPM calculation can have internal Newton functions. It can be interesting to initialize these functions with values proceeding from a previous simulation. For example, for a pipe optimization simulation, the initialize values for the Newton functions can be the results of the previous optimization step.

These values depend on the internal HPM structure. The <Unit Operation> does not have to understand this. An anonymous vector is used to pass the initialization values from step i to step i+1. The <Unit Operation> can not analyze or modify this vector, only store and send it later.

Figure 7 HPM Initialisation

At step 1, the <Unit Operation> sends an empty vector to the <HPM Package> by the "SetInitState" function. The <HPM Package> will used default values to initialize the next "CalcFlow" function. After this function, an anonymous vector is available to the <Unit Operation> through the "GetFinalState" function.

The anonymous vector is used to initialize the next "CalcFlow". The <Unit Operation> send this vector using "SetInitState", if the vector is correct (dimension and values) the <HPM Package> use it for initialize. In no case, the <Unit Operation> have to modify this vector.

# Analysis and design

## Overview

Interfaces are defined to realize the previous Use Cases.

- <HPM Package> implements the following interfaces:

- ICapeHydroFlow

- ICapeHydroFlowRoutine

- ICapeHydroPropertyRoutine

- ICapeHydroPhases

- ICapeHydroFields

- <Unit Operation> implements the following interface:

- ICapeHydroContext

- <HPM Package Manager> implements the following interface:

- ICapeHydroPackageManager

## Sequence diagrams

## Interface diagrams

#### IN- 000: General interface diagram

#### IN- 001: Added interface for HPM Package manager

#### IN- 002: Added interface for Unit Operation

#### IN – 003 Interface for HPM

## State diagrams

## Other diagrams

## Interfaces descriptions

### ICapeHydroFlow

The following methods are exposed by this interface:

- ClearAllProps

- CopyFromHPM

- GetFlowRegime

- GetMixtureProp

- GetSingleHydroProp

- GetTwoHydroProp

- SetGeometryProp

- SetMixtureProp

- SetOverallProp

- SetPresentPhases

- SetSingleHydroProp

- SetSinglePhaseProp

- SetTwoHydroProp

- SetTwoPhaseProp

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | ClearAllProps |
| Returns | CapeError |

Description

Remove all stored property values.

Arguments

| Name | Type | Description |
| --- | --- | --- |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

CearAllProps removes all stored property values that have been set using the SetGeometryProp, SetOverallProp, SetSinglePhaseProp, SetTwoPhaseProp, SetMixtureProp, SetSingleHydroProp, SetTwoHydroProp methods. ClearAllProps does not remove the configuration information in a <HPM Package> .

Using the ClearAllProps methods results in a <HPM Package> that is in the same state as when is was first created.

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | CopyFromHPM |
| Returns | CapeError |

Description

Copies all the stored properties from the <HPM Package> to the current instance of the <HPM Package>.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] source | CapeInterface | Source <HPM Package> from which stored properties will be copied. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | GetFlowRegime |
| Returns | CapeError |

Description

Return main flow regime and qualifier evaluated after a hydrodynamic calculation.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [out] regime | CapeString | The flow regime identifier resulting from the hydrodynamic calculation. This is one of the standards identifiers listed in section 5.4.. |
| [out] qualifier | CapeString | The flow regime qualifier describing more accurately the flow regime identifier. This is one of the standards identifiers listed in section 5.4. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeBadInvOrder – The Flow regime is not available from the <HPM Package>. This exception is raised when the HPM Package is not in the proper state, if the calculation raises an error for example.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | GetMixtureProp |
| Returns | CapeError |

Description

Retrieves flow values for a mixture.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the property which values are requested. |
| [out] values | CapeArrayDouble | Result vector containing property flow value(s) in SI units. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeBadInvOrder – The property required is not available from the <HPM Package>. This exception is raised when the HPM Package is not in the proper state, if the calculation raises an error for example.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | GetSingleHydroProp |
| Returns | CapeError |

Description

Retrieves single-phase flow values for a mixture.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the property which values are requested. |
| [in] phaseLabel | CapeString | Phase label of the phase for which the hydrodynamic property is required. |
| [out] values | CapeArrayDouble | Results vector containing property flow value(s) in SI units. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeBadInvOrder – The property required is not available from the <HPM Package>. This exception is raised when the <HPM Package> is not in the proper state, if the calculation raise an error for example.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | GetTwoHydroProp |
| Returns | CapeError |

Description

Retrieves two-phase flow values for a mixture.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the property which values are requested. |
| [in] phaseLabels | CapeArrayString | List (two values) of Phase labels for which the property is required. |
| [out] values | CapeArrayDouble | Results vector containing property flow value(s) in SI units. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeBadInvOrder – The property required is not available from the <HPM Package>. This exception is raised when the <HPM Package> is not in the proper state, if the calculation raise an error for example.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | SetGeometryProp |
| Returns | CapeError |

Description

Set geometry values.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the geometry property which values are set. |
| [in] values | CapeArrayDouble | Values to set for the property. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | SetMixtureProp |
| Returns | CapeError |

Description

Set flow values for a mixture.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the property which values are set. |
| [in] values | CapeArrayDouble | Values to set for the property. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | SetOverallProp |
| Returns | CapeError |

Description

Set thermodynamic values for the overall mixture.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the property which values are set. |
| [in] values | CapeArrayDouble | Values to set for the property. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | SetPresentPhases |
| Returns | CapeError |

Description

Set hydrodynamic present phases in the flow.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] phaseLabels | CapeArrayString | The list of Phase labels for the Phases present. |
| [in] stateOfAggregation | CapeArrayString | Array of hydrodynamic state of aggregation corresponding to each of the Phase labels. See description below. |
| [in] typeInfo | CapeArrayString | Array of hydrodynamic type of Liquid or Solid corresponding to each of the Phase labels. See description below. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

For HPM, the phase description is quite different of the phase information describe in the [Thermodynamic  and Physical Properties v1.1] document. A correspondence table which associate a thermodynamic to a hydrodynamic phase description has been defined in the 2.3.2 section.

For HPM, the list of supported phase descriptions is defined in the following table:

| StateOfAggregation | TypeInfo |
| --- | --- |
| Vapor | UNDEFINED |
| Liquid | Organic Aqueous Hydrate UNDEFINED |
| Solid | PureSolid SolidSolution Hydrate UNDEFINED |
| Unknown | UNDEFINED |

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | SetSingleHydroProp |
| Returns | CapeError |

Description

Set single-phase hydrodynamic property values for a mixture.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the property which values are set. |
| [in] phaseLabel | CapeString | Phase label for which the property is set. The phase label must be one of the strings used in the SetPresentPhases method of this interface. |
| [in] values | CapeArrayDouble | Values to set for the property. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeFailedInitialisation – The pre-requisites are not valid. The phase referenced has not been created using SetPresentPhases.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | SetSinglePhaseProp |
| Returns | CapeError |

Description

Set single-phase thermodynamic property values for a mixture.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the property which values are set. |
| [in] phaseLabel | CapeString | Phase label for which the property is set. The phase label must be one of the strings used in the SetPresentPhases method of this interface. |
| [in] values | CapeArrayDouble | Values to set for the property. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeFailedInitialisation – The pre-requisites are not valid. The phase referenced has not been created using SetPresentPhases.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The basis is not defined in the set thermodynamic property function. We expect that thermodynamic values are set in the mass basis. The <Unit Operation> is in charge to ask thermodynamic properties in the correct basis before set to the HPM.

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | SetTwoHydroProp |
| Returns | CapeError |

Description

Set two-phase hydrodynamic property values for a mixture.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the property which values are set. |
| [in] phaseLabel | CapeArrayString | Phase label for which the property is set. The phase label must be one of the strings used in the SetPresentPhases method of this interface. |
| [in] values | CapeArrayDouble | Values to set for the property. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeFailedInitialisation – The pre-requisites are not valid. The phase referenced has not been created using SetPresentPhases.

ECapeInvalidArgument – To be used when an invalid argument value is passed. Il would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlow |
| --- | --- |
| Method Name | SetTwoPhaseProp |
| Returns | CapeError |

Description

Set two-phase thermodynamic values for a mixture.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the property which values are set. |
| [in] phaseLabels | CapeArrayString | Phase label for which the property is set. The phase label must be one of the strings used in the SetPresentPhases method of this interface. |
| [in] values | CapeArrayDouble | Values to set for the property. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeFailedInitialisation – The pre-requisites are not valid. The phase referenced has not been created using SetPresentPhases.

ECapeInvalidArgument – To be used when an invalid argument value is passed. Il would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The basis is not defined in the set thermodynamic property function. We expect that thermodynamic values are set in the mass basis. The <Unit Operation> is in charge to ask thermodynamic properties in the correct basis before set to the <HPM Package> .

### ICapeHydroFlowRoutine

The following methods are exposed by this interface:

- CalcFlow

- CheckCalcFlowSpec

- GetFinalState

- SetInitState

| Interface Name | ICapeHydroFlowRoutine |
| --- | --- |
| Method Name | CalcFlow |
| Returns | CapeError |

Description

The Calculate method is used to compute the standard hydrodynamic properties: pressure drop, phase velocities and phase holdups.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] specification1 | CapeString | First specification identifier for hydrodynamic calculation. below for details. |
| [in] specification2 | CapeString | Second specification identifier for hydrodynamic calculation. below for details. |
| [in] flowRegime | CapeString | Specified flow regime identifier for calculation. May be UNDEFINED if no flow regime is specified for calculation. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeFailedInitialisation – The pre-requisites for Flow Calculation are not valid. For example:

- The present phases in the mixture have not been send to the <HPM Package>.

- The geometry of the <Unit Operation> is not send to the <HPM Package>.

- Any other necessary property is not available.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeLicenceError – The operation can not be completed because the licence agreement is not respected.

ECapeSolvingError – The Flow Calculation could not be solved. For example if the solver has run out iterations, or has converged to a trivial solution.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The specification1 and specification2 arguments provide the information necessary to retrieve the values of specifications for flow calculation. The CheckCalculateFlowSpec can be used to check supported specifications. Each specification label contains a sequence of strings in the order defined in the following table:

| Hydrodynamic calculation | specification1 | specification2 | Comments |
| --- | --- | --- | --- |
|  | Us | UNDEFINED | with input phase superficial velocities |
|  | Rk | Um | with input phase volume fractions and mixture velocity |
|  | Rk | Vm | with input phase volume fractions and barycentric velocity |

The list of the input supported flow regime attributes for calculation is defined in the following table:

| Regime for hydrodynamic calculation | flowRegime | Comments |
| --- | --- | --- |
|  | Separated | The gas and liquid phase flow separately. |
|  | Slug | Gas bubbles flow separated by liquid slug. |
|  | Bubble | Small gas bubbles transported by liquid phase. |
|  | UNDEFINED | No impose flow regime in the flow calculation. |

The complete flow regime definition is provided in section 5.4.

| Interface Name | ICapeHydroFlowRoutine |
| --- | --- |
| Method Name | CheckCalcFlowSpec |
| Returns | CapeError |

Description

Check if the <HPM Package> supports the specified type of calculation.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] specification1 | CapeString | First specification identifier for hydrodynamic calculation. |
| [in] specification2 | CapeString | Second specification identifier for hydrodynamic calculation. |
| [in] flowRegime | CapeString | Specified flow regime identifier for calculation. May be UNDEFINED if no flow regime is specified for calculation. |
| [out] isSupported | CapeBoolean | Set to True if the specifications are supported by the Package, False is not supported. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. Il would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The meaning of the specification1, specification2 and flowRegime arguments is the same as for the CalcFlow method in the interface.

If a flowRegime is specified, the CheckCalcFlowSpec method can return a non supported method value. This means that the specified CalcFlow method can not be invoked within this specific context. Within an UNDEFINED flowRegime value, the same CalcFlow method can be valid.

| Interface Name | ICapeHydroFlowRoutine |
| --- | --- |
| Method Name | GetFinalState |
| Returns | CapeError |

Description

Get the anonymous vector resulting of the previous CalcFlow function.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [out] propvals | CapeArrayVariant | Anonymous vector containing the CalcFlow final state values. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidOPeration – To be used when this operation is not valid in the current context, for example if the CalcFlow function failed.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFlowRoutine |
| --- | --- |
| Method Name | SetInitState |
| Returns | CapeError |

Description

Set an anonymous vector to the <HPM Package> to perform a CalcFlow initialization. If the vector is empty, default values initialization will be used.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] propvals | CapeArrayVariant | Anonymous vector containing the CalcFlow initial state values. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeFailedInitialisation – To be used if the initialisation failed with the anonymous vector.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

### ICapeHydroPhases

The following methods 	are exposed by this interface:

- GetNumPhases

- GetPhaseInfo

| Interface Name | ICapeHydroPhases |
| --- | --- |
| Method Name | GetNumPhases |
| Returns | CapeError |

Description

Returns the number of supported phases.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] numberOfPhase | CapeLong | The number of Phases supported. Depending of the return value, the interface is considered as a all Phases or a restricted Phases HPM (see below). |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

A HPM can accept either a restricted number of phases either all phases depending of the presence of an internal mixing rules function or not.

If the return value is equal to zero, the HPM accept all thermodynamic; in other case, the HPM is restricted HPM. In that case, the accepted define phases can be identified using the GetPhaseInfo method.

| Interface Name | ICapeHydroPhase |
| --- | --- |
| Method Name | GetPhaseInfo |
| Returns | CapeError |

Description

Return the Phase description.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] numPhase | CapeLong | A Phase ID. The numPhase value must be in the range 0..GetNumPhases (see below) |
| [out] stateOfAggregation | CapeString | The state of aggregation for the corresponding Phase ID. |
| [out] typeInformation | CapeString | The type information if the state of aggregation value is "Liquid" or "Solid" |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeOutOfBounds – The numPhase value is not in the range [1,numberOfPhase].

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

numPhase must be in the range one to the value returned by the GetNumPhases methods.

In case of GetNumPhases return a null value, the method can be called with a numPhase zero value. The stateOfAggregation and typeInfo must be return with UNDEFINED.

In other case, the stateOfAggregation and typeInformation must return a value from the following table:

| StateOfAggregation | TypeInformation |
| --- | --- |
| Vapor | UNDEFINED |
| Liquid | Organic Aqueous UNDEFINED |
| Solid | PureSolid SolidSolution Hydrate UNDEFINED |
| Unknown | UNDEFINED |

### ICapeHydroPropertyRoutine

The following methods are exposed by this interface:

- CalcSingleHydroProp

- CalcTwoHydroProp

- CheckSingleHydroPropList

- CheckTwoHydroPropList

- GetGeometryPropList

- GetSingleHydroPropList

- GetSinglePhasePropList

- GetTwoPhaseProplist

| Interface Name | ICapeHydroPropertyRoutine |
| --- | --- |
| Method Name | CalcSingleHydroProp |
| Returns | CapeError |

Description

Calculate single-phase flow properties delivered by the HPM after performing a Flow Calculation.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] props | CapeArrayString | The list of identifiers for the single-phase properties to be calculated. See sections 5.6.2 for the standard identifiers. |
| [in] phaseLabel | CapeString | Phase label of the Phase for which the properties are to be calculated. The Phase label must be one of the label strings set in the SetPresentPhases on the ICapeHydroFlow interface. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeSolvingError – One of the property calculations has failed. The request of this property will return an UNDEFINED value.

ECapeLimitedImpl – At least one or more of the requested properties cannot be returned because the calculation (of the particular property) is not implemented.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example for an unrecognised value for the phaseLabel.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

If a client use multiple properties in a call and a ECapeLimitedImpl is returned, then the whole call should be considered to have failed. This implies that no value should be request because the call not respect the HPM validation.

If a ECapeInvalidArgument is returned, we consider not whole call should have failed. The client can request for properties values. For values which calculation failed, the <HPM Package> should returned an UNDEFINED value.

| Interface Name | ICapeHydroPropertyRoutine |
| --- | --- |
| Method Name | CalcTwoHydroProp |
| Returns | CapeError |

Description

Calculate two-phase flow properties delivered by the <HPM Package> after performing a Flow Calculation.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] props | CapeArrayString | The list of identifiers for the two-phase properties to be calculated. See sections 5.6.3 for the standard identifiers. |
| [in] phaseLabels | CapeArrayString | Phase labels of the Phases for which the properties are to be calculated. The Phase labels must be two of the label strings set in the SetPresentPhases on the ICapeHydroFlow interface. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeSolvingError – One of the property calculation has failed.

ECapeLimitedImpl – At least one or more of the requested properties cannot be returned because the calculation (of the particular property) is not implemented.

ECapeInvalidArgument – To be used when an invalid argument value is passed. It would be raised, for example for an unrecognised value for the phaseLabels.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

If a client use multiple properties in a call and a ECapeLimitedImpl is returned, then the whole call should be considered to have failed. This implies that no value should be request because the call not respect the HPM validation.

If a ECapeInvalidArgument is returned, we consider not whole call should have failed. The client can request for properties values. For values which calculation failed, the <HPM Package> should returned an UNDEFINED value.

| Interface Name | ICapeHydroPropertyRoutine |
| --- | --- |
| Method Name | CheckSingleHydroPropList |
| Returns | CapeError |

Description

Return the list of single-phase flow properties which can delivered after performing the specified Flow Calculation.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] specification1 | CapeString | First specification identifier for hydrodynamic calculation. See below for details. |
| [in] specification2 | CapeString | Second specification identifier for hydrodynamic calculation. See below for details. |
| [out] props | CapeArrayString | The list of the single-phase property identifiers from hydro specification necessary for performing the calculation. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. Il would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The meaning of the specification1 and specification2 arguments is the same as for the CalcFlow method in the ICapeHydroFlowRoutine interface.

The property identifiers return by this method belong to the list defined in section 5.6.2.

If <HPM Package> not delivers additional properties, the returned array is empty.

| Interface Name | ICapeHydroPropertyRoutine |
| --- | --- |
| Method Name | CheckTwoHydroPropList |
| Returns | CapeError |

Description

Return the two-phase flow property which can delivered after performing the specified Flow Calculation.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] specification1 | CapeString | First specification identifier for hydrodynamic calculation. See below for details. |
| [in] specification2 | CapeString | Second specification identifier for hydrodynamic calculation. See below for details. |
| [out] props | CapeArrayString | The list of the two-phase property identifiers from hydro specification necessary for performing the calculation. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. Il would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The meaning of the specification1 and specification2 arguments is the same as for the CalcFlow method in the ICapeHydroFlowRoutine interface.

The property identifiers return by this method belong to the list defined in section 5.6.3.

If <HPM Package> not delivers additional properties, the returned array is empty.

| Interface Name | ICapeHydroPropertyRoutine |
| --- | --- |
| Method Name | GetGeometryPropList |
| Returns | CapeError |

Description

Return the list of the needed input geometry properties for performing the specified calculation.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] specification1 | CapeString | First specification identifier for hydrodynamic calculation. See below for details. |
| [in] specification2 | CapeString | Second specification identifier for hydrodynamic calculation. See below for details. |
| [out] props | CapeArrayString | The list of the geometry property identifiers necessary for performing the calculation. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. Il would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The meaning of the specification1 and specification2 arguments is the same as for the CalcFlow method in the ICapeHydroFlowRoutine interface.

The geometry identifiers returned by this method belong to the list defined in section 5.3 of this document.

| Interface Name | ICapeHydroPropertyRoutine |
| --- | --- |
| Method Name | GetSingleHydroPropList |
| Returns | CapeError |

Description

Return the list of single-hydrodynamic property for performing the specified calculation.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] specification1 | CapeString | First specification identifier for hydrodynamic calculation. See below for details. |
| [in] specification2 | CapeString | Second specification identifier for hydrodynamic calculation. See below for details. |
| [out] props | CapeArrayString | The list of the single-property identifiers from hydro specification necessary for performing the calculation. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. Il would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The meaning of the specification1 and specification2 arguments is the same as for the CalcFlow method in the ICapeHydroFlowRoutine interface.

The hydro identifiers return by this method belong to the list defined in section 5.6.2 of this document.

| Interface Name | ICapeHydroPropertyRoutine |
| --- | --- |
| Method Name | GetSinglePhasePropList |
| Returns | CapeError |

Description

Return the list of single-phase thermodynamic property for performing the specified calculation.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] specification1 | CapeString | First specification identifier for hydrodynamic calculation. See below for details. |
| [in] specification2 | CapeString | Second specification identifier for hydrodynamic calculation. See below for details. |
| [out] props | CapeArrayString | The list of the single-phase property identifiers from thermo specification necessary for performing the calculation. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. Il would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The meaning of the specification1 and specification2 arguments is the same as for the CalcFlow method in the ICapeHydroFlowRoutine interface.

The thermo identifiers return by this method belong to the list defined in section 5.5.1 which is part of the "Thermodynamic and Physical Properties – Version 1.1" document.

| Interface Name | ICapeHydroPropertyRoutine |
| --- | --- |
| Method Name | GetTwoPhasePropList |
| Returns | CapeError |

Description

Return the list of two-phase thermodynamic property for performing the specified calculation.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] specification1 | CapeString | First specification identifier for hydrodynamic calculation. See below for details. |
| [in] specification2 | CapeString | Second specification identifier for hydrodynamic calculation. See below for details. |
| [out] props | CapeArrayString | The list of the two-phase property identifiers from thermo specification necessary for performing the calculation. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeInvalidArgument – To be used when an invalid argument value is passed. Il would be raised, for example, if a specification identifier does not belong to the list of recognised identifiers.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The meaning of the specification1 and specification2 arguments is the same as for the CalcFlow method in the ICapeHydroFlowRoutine interface.

The thermo identifiers return by this method belong to the list defined in section 5.5.2 which is part of the "Thermodynamic and Physical Properties – Version 1.1" document.

### ICapeHydroFields

Layer and Field are used to characterize more accuracy flow configurations.

The following methods are described in this section:

- GetFieldInfo

- GetLayerInfo

- GetPresentFields

- GetPresentLayers

- GetPresentPhases

- GetSingleFieldProp

- GetSingleLayerProp

- GetSinglePhaseProp

- GetTwoLayerProp

| Interface Name | ICapeHydroFields |
| --- | --- |
| Method Name | GetFieldInfo |
| Returns | CapeError |

Description

Retrieves the field information for the defined layer.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] layerLabel | CapeString | A (single) Layer label. This must be one of the values return by the GetPresentLayers method. |
| [in] fieldLabel | CapeString | A (single) Field label. This must be one of the values return by the GetPresentFields method. |
| [in] fieldAttribute | CapeString | One of the Field attribute identifiers from the table below. |
| [out] value | CapeString | The value corresponding to the Field attribute identifier – see table below. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Field attribute identifier | Supported values |
| --- | --- |
| EmulsionState | One of the following strings: Yes No Unknown |
| ContinuousPhase | One of the phase label returned in the GetPresentPhases method for the current Field label. |

| Interface Name | ICapeHydroFields |
| --- | --- |
| Method Name | GetLayerInfo |
| Returns | CapeError |

Description

Retrieves the layer information.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] layerLabel | CapeString | A (single) Layer label. This must be one of the values return by the GetPresentLayers method. |
| [in] layerAttribute | CapeString | One of the Layer attribute identifiers from the table below. |
| [out] value | CapeString | The value corresponding to the Field attribute identifier – see table below. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Layer attribute identifier | Supported values |
| --- | --- |
| Pattern | The main flow pattern associated with the Layer. This must be one of the following strings: Continuous Slug Pocket Unknown |
| Pattern2 | Additional pattern for continuous Layer. This must be one of the following strings: Annular Stratified Unknown |
| DispersedState | One of the following strings: Yes No Unknown |
| ContinuousField | One of the Fields defined in the GetPresentFields method for the current Layer label |

For slug flow regimes within a pipe element, "Pocket" term means the layer is in the stratified part of the flow whereas "Slug" term means the layer is within the liquid plug. For other flow regimes, layers are "Continuous" as the composition of the flow mixture is continuous within the pipe element.

| Interface Name | ICapeHydroFields |
| --- | --- |
| Method Name | GetPresentFields |
| Returns | CapeError |

Description

Retrieves the present fields labels in the define layer.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] layerLabel | CapeString | A Layer label. This must be one of the values returned by the GetPresentLayers method. If the Layer label is "Unknown" all the Field labels present in the current flow are returned. |
| [out] fieldLabels | CapeArrayString | The Field labels associated to the Layer label define |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFields |
| --- | --- |
| Method Name | GetPresentLayers |
| Returns | CapeError |

Description

Retrieves the present layers labels in the current flow.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [out] layerLabels | CapeArrayString | The list of Layer labels present in the flow |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFields |
| --- | --- |
| Method Name | GetPresentPhases |
| Returns | CapeError |

Description

Retrieves the present phases labels in the defined field.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] layerLabel | CapeString | A (single) Layer label. This must be one of the values return by the GetPresentLayers method |
| [in] fieldLabel | CapeString | A (single) Field label. This must be one of the values return by the GetPresentFields method |
| [out] phaseLabels | CapeArrayString | The list of the thermodynamic phases present in the Field. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

A same Phase label can be present in multiple Fields.

| Interface Name | ICapeHydroFields |
| --- | --- |
| Method Name | GetSingleFieldProp |
| Returns | CapeError |

Description

Retrieves single-field property value for a flow.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the Field property for which value is requested. This must be one of the single-field Property. The identifiers are listed in the section 5.6.3. |
| [in] layerLabel | CapeString | A (single) Layer label. This must be one of the values return by the GetPresentLayers method. |
| [in] fieldLabel | CapeString | A (single) Field label. This must be one of the values return by the GetPresentFields method. |
| [out] result | CapeDouble | Result value containing the Field property value in SI Units. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFields |
| --- | --- |
| Method Name | GetSingleLayerProp |
| Returns | CapeError |

Description

Retrieves single-layer property value for a flow.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the Layer property for which value is requested. This must be one of the single-layer Property. The identifiers are listed in the section 5.6.1. |
| [in] layerLabel | CapeString | A (single) Layer label. This must be one of the values return by the GetPresentLayers method |
| [out] result | CapeDouble | Result value containing the Layer property value in SI Units. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

| Interface Name | ICapeHydroFields |
| --- | --- |
| Method Name | GetSinglePhaseProp |
| Returns | CapeError |

Description

Retrieves single-phase in a field property value for a flow.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the Field property for which value is requested. This must be one of the single-field Property. The identifiers are listed in the section 5.6.3. |
| [in] layerLabel | CapeString | A (single) Layer label. This must be one of the values return by the GetPresentLayers method. |
| [in] fieldLabel | CapeString | A (single) Field label. This must be one of the values return by the GetPresentFields method. |
| [in] phaseLabel | CapeString | A (single) Phase label. This must be one of the values return by the GetPresentLabels method. |
| [out] result | CapeDouble | Result value containing the Field property value in SI Units. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

.

| Interface Name | ICapeHydroFields |
| --- | --- |
| Method Name | GetTwoLayerProp |
| Returns | CapeError |

Description

Retrieves interface layers property value for a flow.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] property | CapeString | The identifier of the Layer property for which value is requested. This must be one of the single-layer Property. The identifiers are listed in the section 5.6.2. |
| [in] layerLabels | CapeArrayString | List of the Layer labels. The Layer labels must be two of the identifiers returned by the GetPresentLayers method |
| [out] result | CapeDouble | Result value containing the interface Layer property value in SI Units. If the two Layers have no interface, the null value must be returned |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

### ICapeHydroContext

This interface should be implemented by all Unit Operation component that need an ICapeHydroFlow interface in order to get the flowing properties. The following methods are described in this section:

- SetHPM

| Interface Name | ICapeHydroContext |
| --- | --- |
| Method Name | SetHPM |
| Returns | CapeError |

Description

Allows the client of a component (<Unit Operation>) that implements this interface to pass an ICapeHydroFlow interface to the component, so that it can access to the properties of a <HPM Package>.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] hpm | CapeInterface | The <HPM Package> interface |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

### ICapeHydroPackageManager

The ICapeHydroPackageManager interface should only be implemented by a <HPM Package Manager> component. The interface is used to access all the <HPM Package> managed by such a component.

The following methods are described in this section:

- GetHPMPackageList

- GetHPMPackage

| Interface Name | ICapeHydroPackageManager |
| --- | --- |
| Method Name | GetHPMPackageList |
| Returns | CapeError |

Description

Retrieves the names of the <HPM Package> being managed by a <HPM Manager>.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [out] hpmPackages | CapeArrayString | The names of the managed <HPM Package>. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

If no packages are managed, UNDEFINED should be returned.

| Interface Name | ICapeHydroPackageManager |
| --- | --- |
| Method Name | GetHPMPackage |
| Returns | CapeError |

Description

Creates a new instance of a <HPM Package> with the configuration specified by the HPMName argument.

Arguments

| Name | Type | Description |
| --- | --- | --- |
| [in] HPMName | CapeString | The name of one of the <HPM Package> managed by the <HPM Package Manager>. |
| [out] HPMPackage | CapeInterface | The ICapeHydroFlow interface of the named <HPM Package>. |

Errors

ECapeNoImpl – The operation is "not" implemented even if this method can be called for reasons of compatibility with the standards. That is to say that the operation exists, but is not supported by the current implementation.

ECapeFailedInitialisation – This error should be returned if the <HPM Package> cannot be created for any reason.

ECapeUnknown – The error to be raised when other error(s), specified for the operation, are not suitable.

Notes

The <HPM Package Manager> is only a mechanism to create a <HPM Package>. After the <HPM Package> has been created, the <HPM Package Manager> can be destroyed, and this will not affect the normal behaviour of <HPM Packages>.

## Scenarios

# Interface specifications

## Interface IID and registration under the appropriate categrory

From the reserved uuid number we propose for interfaces defined in the following hydro interface identifiers:

/**************************************

Identification Interfaces

**************************************/

#define ICapeHydroContext_IID	678c09cf-7d66-11d2-a67d-00105a42887f

#define ICapeHydroPackageManager_IID	678c09d0-7d66-11d2-a67d-00105a42887f

#define ICapeHydroPhases_IID	678c09d1-7d66-11d2-a67d-00105a42887f

#define ICapeHydroFlowRoutine_IID	678c09d2-7d66-11d2-a67d-00105a42887f

#define ICapeHydroPropertyRoutine_IID	678c09d3-7d66-11d2-a67d-00105a42887f

#define ICapeHydroFields_IID	678c09d4-7d66-11d2-a67d-00105a42887f

#define ICapeHydroFlow_IID 678c09d5-7d66-11d2-a67d-00105a42887f

/**************************************

Categories

**************************************/

// CAPE-OPEN Category

#define CapeOpenComponent_CATID  678c09a1-7d66-11d2-a67d-00105a42887f

#define CapeOpenHydroPackage_CATID  678c09d7-7d66-11d2-a67d-00105a42887f

For example in C++, the registration should be done by adding in rgs file the code below:

'Implemented Categories'

{

{678C09A1-7D66-11D2-A67D-00105A42887F}

{678C09D7-7D66-11D2-A67D-00105A42887F}

}

CapeDescription

{

val Name = s 'my component name'

val Description = s 'my component descritpion'

val CapeVersion = s 'my component version'

val Categories = s 'my component category'

val ComponentVersion = s 'my component CAPE-OPEN version'

val VendorURL = s 'my company URL'

val HelpURL = s 'my Help URL'

val About = s 'For any information please contact support....'

}

## COM IDL

### Interface ICapeHydroContext : IDispatch

[

uuid(678c09cf-7d66-11d2-a67d-00105a42887f),

helpstring("ICapeHydroContext Interface"),

dual,

pointer_default(unique)

]

interface ICapeHydroContext : IDispatch

{

//Allows the client of a component (<Unit Operation>) that implements this interface to pass an

//ICapeHydroFlow interface to the component, so that it can access to the properties of a HPM.

[id(0)] HRESULT SetHPM([in] IDispatch* hpminterface);

};

### Interface ICapeHydroPackageManager : IDispatch

[

uuid(678c09d0-7d66-11d2-a67d-00105a42887f),

helpstring("ICapeHydroPackageManager Interface"),

dual,

pointer_default(unique)

]

interface ICapeHydroPackageManager : IDispatch

{

//Retrieves the names of the HPM Packages being managed by a HPM Manager component.

[id(0)] HRESULT GetHPMPackageList([out] VARIANT* hpmPackages);

//Creates a new instance of a HPM Package with the configuration specified by the HPMName argument.

[id(1)] HRESULT GetHPMPackage([in] BSTR HPMName, [out]IDispatch**HPMPackage);

};

### Interface ICapeHydroPhases : IDispatch

[

uuid(678c09d1-7d66-11d2-a67d-00105a42887f),

helpstring("ICapeHydroPhases Interface"),

dual,

pointer_default(unique)

]

interface ICapeHydroPhases : IDispatch

{

//Returns the number of supported phases.

[id(0)] HRESULT GetNumPhases([out] LONG* num);

//Return the Phase description

[id(1)] HRESULT GetPhaseInfo([in] LONG numPhase, [out]BSTR* stateOfAggregation, [out]BSTR* typeInfo);

};

### Interface ICapeHydroFlowRoutine : IDispatch

[

uuid(678c09d2-7d66-11d2-a67d-00105a42887f),

helpstring("ICapeHydroFlowRoutine Interface"),

dual,

pointer_default(unique)

]

interface ICapeHydroFlowRoutine : IDispatch

{

//The Calculate method is used to compute the standard hydrodynamic properties: pressure drop, phase velocities and phase holdups.

[id(0)] HRESULT CalcFlow([in] BSTR specification1,[in] BSTR specification2,[in] BSTR flowregime);

//Check if the HPM Package support the specified type of calculation.

[id(1)] HRESULT CheckCalcFlowSpec([in] BSTR specification1,[in] BSTR specification2,[in] BSTR flowregime, [out]VARIANT_BOOL* isSupported);

//Get the anonymous vector resulting of the previous CalcFlow function.

[id(2)] HRESULT GetFinalState([out]VARIANT* propvals);

//Set an anonymous vector to the HPM to perform a CalcFlow initialization. If the vector is empty, default values initialization will be used.

[id(3)] HRESULT SetInitState([in] VARIANT propvals);

};

### Interface ICapeHydroPropertyRoutine : IDispatch

[

uuid(678c09d3-7d66-11d2-a67d-00105a42887f),

helpstring("ICapeHydroPropertyRoutine Interface"),

dual,

pointer_default(unique)

]

interface ICapeHydroPropertyRoutine : IDispatch

{

//Calculate single-phase flow properties delivered by the HPM after performing a Flow Calculation.

[id(0)] HRESULT CalcSingleHydroProp([in] VARIANT props,[in] BSTR phaseLabel);

//Calculate two-phase flow properties delivered by the HPM after performing a Flow Calculation.

[id(1)] HRESULT CalcTwoHydroProp([in] VARIANT props,[in] VARIANT phaseLabels);

//Return the single-phase flow property which can delivered by the HPM after performing the specified Flow Calculation.

[id(2)] HRESULT CheckSingleHydroPropList([in] BSTR specification1,[in] BSTR specification2,[out] VARIANT *props);

//Return the two-phase flow property which can delivered by the HPM after performing the specified Flow Calculation.

[id(3)] HRESULT CheckTwoHydroPropList([in] BSTR specification1,[in] BSTR specification2,[out] VARIANT *props);

//Return the list of the needed input geometry properties for performing the specified calculation.

[id(4)] HRESULT GetGeometryPropList([in] BSTR specification1,[in] BSTR specification2,[out] VARIANT *props);

// Return the list of single-hydrodynamic property for performing the specified calculation

[id(5)] HRESULT GetSingleHydroPropList([in] BSTR specification1,[in] BSTR specification2,[out] VARIANT *props);

//Return the list of single-phase thermodynamic property for performing the specified calculation.

[id(6)] HRESULT GetSinglePhasePropList([in] BSTR specification1,[in] BSTR specification2,[out] VARIANT *props);

//Return the list of two-phase thermodynamic property for performing the specified calculation.

[id(7)] HRESULT GetTwoPhasePropList([in] BSTR specification1,[in] BSTR specification2,[out] VARIANT *props);

};

### Interface ICapeHydroFields : IDispatch

[

uuid(678c09d4-7d66-11d2-a67d-00105a42887f),

helpstring("ICapeHydroFields Interface"),

dual,

pointer_default(unique)

]

interface ICapeHydroFields : IDispatch

{

//

[id(0)] HRESULT GetFieldInfo([in] BSTR layerLabel,[in] BSTR fieldLabel,[in] BSTR fieldAttribute, [out]BSTR* value);

//

[id(1)] HRESULT GetLayerInfo([in] BSTR layerLabel,[in] BSTR fieldAttribute, [out]BSTR* value);

//Retrieves the present fields labels in the define layer.

[id(2)] HRESULT GetPresentFields([in] BSTR layerLabel,[out]VARIANT* fieldLabels);

//Retrieves the present layers labels in the current flow.

[id(3)] HRESULT GetPresentLayers([out]VARIANT* layerLabels);

//Retrieves the present phases labels in the define field.

[id(4)] HRESULT GetPresentPhases([in] BSTR layerLabel,[in] BSTR fieldLabel,[out]VARIANT* phaseLabels);

//Retrieves single-field property value for a flow.

[id(5)] HRESULT GetSingleFieldProp([in] BSTR pty,[in] BSTR layerLabel,[in] BSTR fieldLabel,[out]double* result);

//Retrieves single-layer property value for a flow.

[id(6)] HRESULT GetSingleLayerProp([in] BSTR pty,[in] BSTR layerLabel,[out]double* result);

//Retrieves single-phase in a field property value for a flow.

[id(7)] HRESULT GetSinglePhaseProp([in] BSTR pty,[in] BSTR layerLabel,[in] BSTR fieldLabel,[in] BSTR phaseLabel,[out]double* result);

//Retrieves interface layers property value for a flow.

[id(8)] HRESULT GetTwoLayerProp([in] BSTR pty,[in] VARIANT layerLabels,[out]double* result);

};

### Interface ICapeHydroFlow : IDispatch

[

uuid(678c09d5-7d66-11d2-a67d-00105a42887f),

helpstring("ICapeHydroFlow Interface"),

dual,

pointer_default(unique)

]

interface ICapeHydroFlow : IDispatch

{

//Remove all stored property values

[id(0)] HRESULT ClearAllProps();

//Copies all the stored properties (which have been set using the ICapeHydroFlow::Set... methods) from the HPM to the current instance of the HPM.

[id(1)] HRESULT CopyFromHPM([in]IDispatch*hpmSource);

//Return main flow regime and qualifier evaluated after a hydrodynamic calculation.

[id(2)] HRESULT GetFlowRegime([out]BSTR*regime,[out]BSTR* qualifier);

//Retrieves flow values for a mixture.

[id(3)] HRESULT GetMixtureProp([in]BSTR pty,[out]VARIANT* values);

//Retrieves single-phase flow values for a mixture.

[id(4)] HRESULT GetSingleHydroProp([in]BSTR pty,[in]BSTR phaseLabel,[out]VARIANT* values);

//Retrieves two-phase flow values for a mixture.

[id(5)] HRESULT GetTwoHydroProp([in]BSTR pty,[in]VARIANT phaseLabel,[out]VARIANT* values);

//Set geometry values for a HPM.

[id(6)] HRESULT SetGeometryProp([in]BSTR pty,[in]VARIANT values);

//Set flow values for a mixture.

[id(7)] HRESULT SetMixtureProp([in]BSTR pty,[in]VARIANT values);

//Set thermodynamic values for the overall mixture.

[id(8)] HRESULT SetOverallProp([in]BSTR pty,[out]VARIANT *values);	//Set hydrodynamic present phases in the flow.

[id(9)] HRESULT SetPresentPhases([in]VARIANT phaseLabel,[in]VARIANT stateOfAggregation,[in]VARIANT typeInfo);

//Set single-phase hydrodynamic property values for a mixture.

[id(10)] HRESULT SetSingleHydroProp([in]BSTR pty,[in]BSTR phaseLabel,[in]VARIANT values);

//Set single-phase thermodynamic property values for a mixture.

[id(11)] HRESULT SetSinglePhaseProp([in]BSTR pty,[in]BSTR phaseLabel,[in]VARIANT values);

//Retrieves two-phase hydrodynamic property values for a mixture.

[id(12)] HRESULT SetTwoHydroProp([in]BSTR pty,[in]VARIANT phaseLabel,[in]VARIANT values);

//Retrieves two-phase thermodynamic values for a mixture.	[id(14)] HRESULT SetTwoPhaseProp([in]BSTR pty,[in]VARIANT phaseLabel,[in]VARIANT values);

};

# Notes on the interface specifications

## Convention Note on UNDEFINED

Throughout the document UNDEFINED refers to a NULL BSTR for string arguments. See below how to implement these values in C++ and in VB.

| Type of Data | Declaration and Usage |
| --- | --- |
| BSTR | In C++ Acceptable |
|  | BSTR strArg = NULL; |
|  | Not Acceptable |
|  | BSTR str = ::SysAllocString(L“”); |
|  | In VB Acceptable |
|  | Dim strArg as String strArg = vbNullString |
|  | Not Acceptable |
|  | Dim strArg as String strArg = “” |

## Correspondance thermodynamic phase property – hydrodynamic qualifier

| Thermodynamic |  | Hydrodynamic |  |
| --- | --- | --- | --- |
| StateOfAggregation |  |  |  |
| Vapor |  | Vapor |  |
| StateOfAggregation | KeyCompoundId |  | Type (OfLiquid) |
| Liquid | UNDEFINED | Liquid | Organic |
|  | Water |  | Aqueous |
|  |  |  | UNDEFINED |
| StateOfAggregration | TypeOfSolid |  | Type (OfSolid) |
| Solid | PureSolid | Solid | PureSolid |
|  | SolidSolution |  | SolidSolution |
|  | HydrateI |  | Hydrate |
|  | HydrateII |  | Hydrate |
|  | HydrateH |  | Hydrate |
|  |  |  | UNDEFINED |
| Unknown |  | Unknown |  |

## Geometry identifiers

| Identifier | Meaning | units |
| --- | --- | --- |
| diameter | pipe diameter | m |
| inclination | pipe inclination with horizontal | deg |
| roughness | absolute pipe wall roughness | m |

## Flow regime Identifiers

The flow regime is described in a finite number of identifiers.

| Identifier | Meaning |
| --- | --- |
| Homogeneous | The flow regime is defined by a single or a two liquid phase,. |
| Separated | The gas and the liquid flow separately. |
| Slug | Long bubbles flow separated by liquid slugs. These slugs can contain small bubbles. |
| Bubble | The gas phase is transported by the liquid in the form of small bubbles. |
| UNDEFINED | Not impose flow regime at the beginning calculation – Calculation error |

| Flow regime: Identifier | Flow regime: Qualifier | Meaning |
| --- | --- | --- |
| Homogeneous | SinglePhase | A single phase flow |
|  | TwoPhase | A two liquid (organic and aqueous) phase flow |
|  | Unknown |  |
| Separated | StratifiedSmooth | A linear gas-liquid interface can be observed. |
|  | StratfiedWavy | Some waves appears in the gas-liquid interface. |
|  | Annular | The liquid phase forms a continuous film on the wall and the gas flow at the centre. |
|  | Unknown |  |
| Slug |  |  |
| Bubble |  |  |

## Physical Property identifiers

Physical property identifiers are part of the non-constant single or two-phase mixture properties described in the CAPE-OPEN Open Interface Specifications – Thermodynamics and Physical Properties Version 1.1.

### Single-phase properties

| Identifier | Meaning | units |
| --- | --- | --- |
| density | Density | mol/m3 |
| heatCapacityCp | Heat capacity at constant pressure (Cp) | J/K |
| thermalConductivity | Thermal conductivity | W/(m.K) |
| viscosity | Viscosity | Pa.S |

### Two-phase properties

| Identifier | Meaning | units |
| --- | --- | --- |
| surfaceTension | Interfacial tension for a pair of phases | N/m |

## Hydrodynamic Flow Properties

### Mixture properties

| Identifier | Meaning | units |
| --- | --- | --- |
| pressureDropAcceleration | Acceleration pressure drop | Pa/m |
| pressureDropFrictional | Frictional pressure drop | Pa/m |
| pressureDropGravity | Gravity pressure drop | Pa/m |
| pressureDropTotal | Pressure drop (Acceleration + Frictional + Gravity) | Pa/m |
| waterCut | Percentage of water in the liquid phase fraction. This property is available for emulsion feature. | - |

### Single Phase properties

| Identifier | Meaning | units |
| --- | --- | --- |
| absoluteVelocity | Absolute velocity | m/s |
| effectiveViscosity | Effective viscosity including water/oil/solid dispersion effects | Pa.s |
| superficialVelocity | Superficial velocity | m/s |
| volumeFraction | Volume fraction (hold-up) | - |
| wallShearStress | Mean shear stress for a phase and the wall | Pa |

### Two-phase properties

| Identifier | Meaning | units |
| --- | --- | --- |
| shearStress | Shear stress for a pair of phases | Pa |

## Hydrodynamic Layer and Field properties

### Layer pattern identifiers

| Identifier | Meaning |
| --- | --- |
| Continuous | For Homogeneous, Separated or Bubble flow regimes, a layer is describe as a Continuous layer. |
| Slug | For Slug flow regime, a layer is a Slug if the gas bubble is not present in the layer area. |
| Pocket | For Slug flow regime, a layer is a Pocket if the gas bubble is present in the layer area. |
| UNDEFINED | If the HPM not defines the layers patterns. |

### Single-layer properties

| Identifier | Meaning | units |
| --- | --- | --- |
| diameter | Hydraulic diameter of the layer. | m |
| lengthFraction | Layer length fraction. | - |
| thickness | Thickness for stratified or annulus layer. | m |
| volumeFraction | Layer volume fraction in the flow. The sum of layer volume fractions must be equal to 1. | - |
| wallShearStress | Mean shear stress for a layer and the wall. | Pa |
| waveFrequency | Wave frequency for a stratified layer. |  |
| waveHeight | Wave height for a stratified layer. | m |
| wetPerimeter | Perimeter in contact between the wall and the layer. | m |

### Two-layer properties

| Identifier | Meaning | units |
| --- | --- | --- |
| interfacePerimeter | Perimeter in contact for a pair of Layers. | m |
| interfaceShearStress | Mean shear stress between two layers | Pa |

### Single-field properties

| Identifier | Meaning | units |
| --- | --- | --- |
| absoluteVelocity | Absolute velocity | m/s |
| effectiveViscosity | Effective viscosity | Pa.s |
| superficialVelocity | Superficial velocity | m/s |
| volumeFraction | Field volume fraction in the Layer. In a layer, the sum of the fields volume fractions must be equal to 1. | - |

### Phase properties

| Identifier | Meaning | units |
| --- | --- | --- |
| volumeFraction | Phase volume fraction in the Field. In a field, the sum of the phases volume fractions must be equal to 1. | - |

# Prototype implementation

## Example of flow calculation with phase mixing rule done by <Unit Operation>

...

// After thermodynamic flash calculation

//    4 phaseLabels defined: {"oil", "gas", "water", "hydrate"}

//  and phases properties

//	- US  superficial velocity

//	- RHO density

//	- VF  volume fraction

...

// Phases mixing rule to: Vapor, Liquid, Solid-Hydrate

CapeArrayString hydroLabels(3) = {"gas", "liquid", "solid"};

for (i=0 ; i<numberOfPresentPhases; ++i)

{

thermoMO.GetPhaseInfo(phaseLabels[i], "StateOfAggregation", state);

if (state == "Vapor")

{

hydroUS["gas"]  = thermoUS[phaseLabels[i]];

hydroRHO["gas"] = thermoRHO[phaseLabels[i]] * thermoUS[phaseLabels[i]];

...

} else if (state == "Liquid") {

hydroUS["liquid"]  += thermoUS[phaseLabels[i]];

hydroRHO["liquid"] += thermoRHO[phaseLabels[i]] * thermoUS[phaseLabels[i]];

...

} else if (state == "Solid") {

thermoMO.GetPhaseInfo(phaselabel[i], "TypeOfSolid", type);

if (type == "HydrateI" || type == "HydrateII" || ... ) {

hydroUS["solid"]  += thermoUS[phaseLabels[i]];

hydroRHO["solid"] += thermoRHO[phaseLabels[i]] * thermoUS[phaseLabels[i]];

...

} else {

throw exception;

}

}

}

hydroRHO["liquid"] /= hydroUS["liquid"];

...

// Set the thermodynamic properties to the HPM

hydroHPM.SetPresentPhases(hydroLabels);

hydroHPM.SetPhaseQualifier("gas", "Vapor", "UNDEFINED");

hydroHPM.SetPhaseQualifier("liquid, "Liquid", "UNDEFINED");

hydroHPM.SetPhaseQualifier("solid", "Solid", "Hydrate");

hydroHPM.SetSinglePhaseProp("Density", "gas", hydroRHO["gas"]);

hydroHPM.SetSinglePhaseProp("SuperficialVelocity", "gas", hydroUS["gas"]);

...

// Flow properties calculation

hydroHPM.CalcFlow("Us", "UNDEFINED");

// Get the standard flow properties

hydroHPM.GetSinglePhaseProp("VolumeFraction", "gas", hydroVF["gas"]);

hydroHPM.GetSinglePhaseProp("VolumeFraction", "liquid", hydroVF["liquid"]);

hydroHPM.GetSinglePhaseProp("VolumeFraction", "hydrate", hydroVF["hydrate"]);

// Set the flow properties to corresponding phases

thermoVF["gas"]     = hydroVK["gas"];

thermoVF["oil"]     = hydroVK["liquid"] * thermoUS["oil"]   / hydroUS["liquid"];

thermoVF["water"]   = hydroVK["liquid"] * thermoUS["water"] / hydroUS["liquid"];

thermoVF["hydrate"] = hydroVK["solid"];

## Example of flow calculation with phase mixing rule done internaly in HPM

...

// After thermodynamic flash calculation

//    4 phaseLabels define: {"oil", "gas", "water", "hydrate"}

//  and phases properties

//	- US  superficial velocity

//	- RHO density

//	- VF  volume fraction

...

// Set the phases qualifiers to the Flowing Object

HPM.SetPresentPhases(phaseLabels);

for (i=0 ; i<numberOfPresentPhases; ++i)

{

thermoMO.GetPhaseInfo(phaseLabels[i], "StateOfAggregation", state);

if (state == "Vapor")

{

mainQualifier = "Vapor";

subQualifier  = "UNDEFINED";

} else if (state == "Liquid") {

thermoMO.GetPhaseInfo(phaseLabels[i], "KeyCompoundId", type);

if (type == "Water")

{

mainQualifier = "Liquid";

subQualifier  = "Water";

} else {

mainQualifier = "Liquid";

subQualifier  = "Organic";

}

} else if (state == "Solid") {

thermoMO.GetPhaseInfo(phaseLabels[i], "TypeOfSolid", type);

if (type == "HydrateI" || type == "HydrateII" || ... )

{

mainQualifier = "Solid";

subQualifier  = "Hydrate";

} else {

...

}

}

hydroHPM.SetPhaseQualifier(phaselabel[i], mainQualifier, subQualifier)

}

// Set the thermodynamic properties to the Flowing Object

for (i=0 ; i<numberOfPresentPhases; ++i)

{

phase = phaseLabels[i]

hydroHPM.SetSinglePhaseProp("Density", phase, thermoRHO[phase]);

hydroHPM.SetSinglePhaseProp("SuperficialVelocity", phase, hydroUS[phase]);

...

}

// Flow properties calculation

hydroHPM.CalcFlow("Us", "UNDEFINED");

// Get the flow properties

for (i=0 ; i<numberOfPresentPhases; ++i)

{

hydroHPM.GetSinglePhaseProp("VolumeFraction", phaseLabels[i], thermoVF[phaseLabels[i]]);

...

}

## Example of field retrieve property

// Flow properties calculation

...

hydroHPM.CalcFlow("Us", "UNDEFINED");

...

// Ask for layers and fields present in the flow and some properties

CapeArrayField presentLayers;

hydroHPM.GetPresentLayers(presentLayers);

int numPresentlayers = presentLayers.dim();

for (int i=0; i<numPresentLayers, ++i)

{

CapeArrayString presentFields;

hydroHPM.getPresentFields(presentLayers[i], presentFields);

double layerLengthFraction, layerVolumeFraction;

hydroHPM.getSingleLayerProp("lengthFraction", presentLayers[i], layerLengthFraction);

hydroHPM.getSingleLayerProp("volumeFraction", presentLayers[i], layerVolumeFraction);

int numFields = presentFields.dim();

for (int j=0; j<numFields, ++j)

{

CapeArrayString presentPhases;

hydroHPM.getPresentPhases(presentLayers[i], presentFields[j], presentPhases);

// Ask for the first hydrodynamic phase informations

CapeString stateOfAggregation, typeInfo;

hydroHPM.getPhaseInfo(presentPhases[0], stateOfAggregation, typeInfo);

double fieldAbsoluteVelocity, fieldVolumeFraction;

hydroHPM.getSingleFieldProp("absoluteVelocity", presentLayers[i], presentFields[j], 					fieldAbsoluteVelocity);

hydroHPM.getSingleFieldProp("volumeFraction", presentLayers[i], presentFields[j], 					fieldVolumeFraction);

}

}

# Specific glossary terms

Layer characterizes a flow entity which defines a continuous area in term of composition and properties. A layer is composed by fields. The characterization of the layer and the presence of the layers in the flow are the responsibility of the hydrodynamic module.

- Layer property: composition in term of fields, layer pattern, wall shear stress, ...

- Two layer property: shear stress between present layers, heat exchange coefficient between layers, ...

Field characterizes a flow entity where present phases flow at the same average velocity. A field can be constituted by a mixture of thermodynamic phases (for instance an emulsion: water dispersed in oil), or a pure phase (for instance oil droplets or stratified water phase). The characterization of the field and the presence of these fields in a layer are the responsibility of the hydrodynamic module.

- Field property: composition in term of phase, average velocity, ...

Flow regime characterizes the state of the combination of the present layers.

- Flow parameters are set of properties: phase superficial velocities, phase fractions in cross section and the mixture velocity, phase fractions in cross section and barycentric velocity.

- Flow property for example: composition in term of field content in a cross section, shear stress between present fields, overall pressure drop, hydrostatic head, friction, acceleration pressure drop, heat exchange coefficient between fields and overall heat exchange coefficient with the wall...

Geometry: diameter, slope roughness, radius of curvature, outlet bend slope of the unit operation Pipe.

Hydrodynamic Point Model (HPM) is a model that computes pressure drop, phase holdups, phase velocities, flow regime and possibly other flow variables for a cross section at one point along a pipeline assuming fully developed flow. The "point" – part of the name hence refers to the direction along the pipeline, i.e. normal to the cross section. Current commercial HPMs deliver average values for the cross section or for pipe unit element in case of intermittent flows, but future versions may in addition compute cross sectional distributions of some variables, for instance velocity profiles and concentration profiles.

Hydrodynamic properties

- Absolute velocity

- Superficial velocity correspond to the inlet flow rate of a thermodynamic phase.

Thermodynamic phase is a homogeneous phase which name is qualified by the Physical Property Package.

- Thermodynamic phase property is a non-constant property depend on the state of the thermodynamic phase. A property can depend on the state of one phase, e.g. density, viscosity, enthalpy... , or two phase, e.g. surface tension.

# Bibliography

## Process simulation references

- [1] Gainville M., Roux P., Pons M., Ricordeau A.and Paen D., Integrated Flow Modeling Platform Using CAPE-OPEN Standard, SPE 110864, 2007.

- [2] Paen D., Roux P., Ricordeau A., Vacher A and Gainville M, Upstream simulation lifecycle, ESCAPE 18, 2008.

- [3] Bendiksen K., MalnesD., Moe R. and Nuland S., The Dynamic Two-Fluid Model OLGA: Theory and Application, SPE Production Engineering, May 1991, pp. 171-180.

- [4] Henriot V., Duret E., Heintzé E. and Courbit A., Multiphase Production Control: Application to Slug Flow, Oil&Gas Science and Technology – Rev. IFP Energies Nouvelles, Vol. 57, No. 1, pp.87-98.

## Computing references

- [5] Banks P.S., K Irons and M R Woodman, Interoperability of Process Simulation Software, Oil&Gas Journal, 2005.

## General references

- [6] Conceptual Design Document (CDD2) for CAPE-OPEN Project, CAPE-OPEN, 2000.

- [7] CAPE-OPEN Open Interface Specifications, Thermodynamic and Physical Properties Version 1.1, CAPE-OPEN, 2003.

- [8] CAPE-OPEN Open Interface Specifications, Unit Operations, CAPE-OPEN, 1999.

- [9] Open Interface Specification: Utilities Common Interface, CAPE-OPEN, 2003.

# Appendices

Layer and Field identifiers are defined in [3.6.5 ICapeHydroFields].

One gives some examples of flow patterns (2-phases and 3-phases) defined in term of layers and fields. For a better understanding of the ICapeHydroFields usage, one presents the way to obtain layer and field properties within the package interfaces for the 2-phases flow example.

In the schemes, thermodynamic phases are defined with the corresponding table: white-gas, blue-liquid, yellow-water.

## Example of 2-phase flow patterns defined with Layers and Fields

|  | Present phases |
| --- | --- |
|  | water gas |
| Layers | Fields in layer |
| Label layer_1 Volume fraction value |  |
| Field labels field_1 Stratified ¦ Annular Annular Slug ¦ Pocket ¦ Continuous Continuous Dispersed No Continuous field field_1 Length Fraction 1 Wet perimeter Pi . Diameter | Label field_1 Volume fraction value |
|  | Phase labels water Phase volume fraction value [1] Absolute Velocity value Emulsion No Continuous phase water |
| Label layer_2 Volume fraction value |  |
| Field labels field_2 Stratified ¦ Annular Annular Slug ¦ Pocket ¦ Continuous Continuous Dispersed No Continuous field field_2 Length Fraction 1 Wet perimeter 0 | Label field_2 Volume fraction value |
|  | Phase labels gas Phase volume fraction value [1] Absolute Velocity value Emulsion No Continuous phase gas |
| Labels layer_1 & layer_2 Interface perimeter value |  |

Figure 8 Annular flow regime defined by layers and fields

// In functions, one use the following conventions:

// input values: Standard

// returned values: Italic

// asked property or attribute: "Name"

// Ask for present layers

hydroHPM.GetPresentLayers(layerLabels);

// Ask for first layer properties & attributes

hydroHPM.GetSingleLayerProp("VolumeFraction", layerLabels[0], layerVF);

hydroHPM.GetSingleLayerProp("LengthFraction", layerLabels[0], lengthFraction);

hydroHPM.GetSingleLayerProp("WetPerimeter",   layerLabels[0], wetPerimeter  );

hydroHPM.GetLayerInfo(layerLabels[0], "Pattern",        layerPattern);

hydroHPM.GetLayerInfo(layerLabels[0], "Pattern2",       isStratified);

hydroHPM.GetLayerInfo(layerLabels[0], "DispersedState", isDispersed);

// Ask for layers interface properties

hydroHPM.GetTwoLayerProp("InterfacePerimeter", layerLabels[0:1], perimeter);

// Ask for present fields in first layer

hydroHPM.getPresentFields(layerLabels[0], fieldLabels);

hydroHPM.getLayerInfo(layerLabels[0], "ContinuousField", continuousFieldLabel);

// Ask for first field - first layer properties & attributes

hydroHPM.GetSingleFieldProp("VolumeFraction",   layerLabels[0], fieldLabels[0], fieldVF);

hydroHPM.GetSingleFieldProp("AbsoluteVelocity", layerLabels[0], fieldLabels[0], fieldV);

hydroHPM.GetFieldInfo(layerLabels[0], ], fieldLabels[0], "EmulsionState", isEmulsion);

// Ask for present phase in first field - first layer

hydroHPM.GetPresentPhases(layerLabels[0], ], fieldLabels[0], phaseLabels);

hydroHPM.GetFieldInfo(layerLabels[0], ], fieldLabels[0], "ContinuousPhase", continuous);

// Ask for first phase - first field - first layer properties

hydroHPM.GetSinglePhaseProp("VolumeFraction", layerLabels[0], fieldLabels[0], phaseLabels(0], phaseVF);

## Examples of 3-phase flow patterns defined with Layers and Fields

|  | Present phases |
| --- | --- |
|  | water liquid gas |
| Layers | Fields in layer |
| Label layer_1 Volume fraction value |  |
| Field labels field_1 – field_2 Stratified ¦ Annular Stratified Slug ¦ Pocket ¦ Continuous Pocket Dispersed Yes Continuous field field_1 Length Fraction value Interface perimeters values [2] Wet perimeter value | Label field_1 Volume fraction value |
|  | Phase labels water Phase volume fraction value [1] Velocity value Emulsion No Continuous phase water |
|  | Label field_2 Volume fraction value |
|  | Phase labels liquid Phase volume fraction value [1] Velocity value Emulsion No Continuous phase liquid |
| Label layer_2 Volume fraction value |  |
| Field labels field_3 Stratified ¦ Annular Stratified Slug ¦ Pocket ¦ Continuous Pocket Dispersed No Continuous field field_3 Length Fraction 1 Interface perimeters values [2] Wet perimeter value | Label field_3 Volume fraction value |
|  | Phase labels gas Phase volume fraction value [1] Velocity value Emulsion No Continuous phase gas |
| Label layer_3 Volume fraction value |  |
| Field labels field_4 – field_5 Stratified ¦ Annular Stratified Slug ¦ Pocket ¦ Continuous Slug Dispersed No Continuous field field_4 Length Fraction 1 Interface perimeters values [2] Wet perimeter value | Label field_4 Volume fraction value |
|  | Phase labels water Phase volume fraction value [1] Velocity value Emulsion No Continuous phase water |
|  | Label field_5 Volume fraction value |
|  | Phase labels liquid Phase volume fraction value [1] Velocity value Emulsion No Continuous phase liquid |
| Labels layer_1 & layer_2 Interface perimeter value Labels layer_1 & layer_3 Interface perimeter 0. Labels layer_2 & layer_3 Interface perimeter 0. |  |

Figure 9 Slug flow regime defined by layers and fields

|  | Present phases |
| --- | --- |
|  | water liquid gas |
| Layers | Fields in layer |
| Label layer_1 Volume fraction value |  |
| Field labels field_1 Stratified ¦ Annular Stratified Slug ¦ Pocket ¦ Continuous Pocket Dispersed No Continuous field field_1 Length Fraction value Interface perimeters values [1] Wet perimeter value | Label field_1 Volume fraction value |
|  | Phase labels water Phase volume fraction value [1] Velocity value Emulsion No Continuous phase water |
| Label layer_2 Volume fraction value |  |
| Field labels field_2 Stratified ¦ Annular Stratified Slug ¦ Pocket ¦ Continuous Pocket Dispersed No Continuous field field_2 Length Fraction value Interface perimeters values [2] Wet perimeter 0 | Label field_2 Volume fraction value |
|  | Phase labels liquid Phase volume fraction value [1] Velocity value Emulsion No Continuous phase liquid |
| Label layer_3 Volume fraction value |  |
| Field labels field_3 Stratified ¦ Annular Stratified Slug ¦ Pocket ¦ Continuous Pocket Dispersed No Continuous field field_3 Length Fraction value Interface perimeters values [2] Wet perimeter value | Label field_3 Volume fraction value |
|  | Phase labels gas Phase volume fraction value [1] Velocity value Emulsion No Continuous phase gas |
| Label layer_4 Volume fraction value |  |
| Field labels field_4 Stratified ¦ Annular Stratified Slug ¦ Pocket ¦ Continuous Slug Dispersed No Continuous field field_4 Length Fraction value Interface perimeters values [1] Wet perimeter value | Label field_4 Volume fraction value |
|  | Phase labels water Phase volume fraction value [1] Velocity value Emulsion No Continuous phase water |
| Label layer_5 Volume fraction value |  |
| Field labels field_5 – field_6 Stratified ¦ Annular Stratified Slug ¦ Pocket ¦ Continuous Slug Dispersed No Continuous field field_5 Length Fraction value Interface perimeters values [1] Wet perimeter value | Label field_5 Volume fraction value |
|  | Phase labels liquid Phase volume fraction value [1] Velocity value Emulsion No Continuous phase liquid |
|  | Label field_6 Volume fraction value |
|  | Phase labels gas Phase volume fraction value [1] Velocity value Emulsion No Continuous phase gas |
| Labels layer_1 & layer_2 Interface perimeter value Labels layer_1 & layer_3 Interface perimeter 0. Labels layer_2 & layer_3 Interface perimeter value ... |  |

Figure 10 Slug flow regime with a stratified bed defined by layers and fields

The configuration user interface may be a stand-alone application which is part of the <HPM Package> (ref. for thermodynamics package: CAPE-OPEN Conceptual Design Document, 8.4.1.3)

The "Edit" and "GetParameters" methods in the ICapeUtilities interface [9]. The "GetParameters" method returns an ICapeCollection interface. This interface contain a collection of parameters which may be modified by the <End User>.

The layers and fields labels define more accurately a flow configuration than flow regime identifier. The used labels are unique and constant along the flow calculations.

"GetParameters" method is defined in the ICapeUtilies interface[9].